Continuous flow organocatalyzed methoxycarbonylation of benzyl alcohol derivatives with dimethyl carbonate

Thomas Toupy†, Loïc Bovy†, and Jean-Christophe M. Monbaliu*,†

†Center for Integrated Technology and Organic Synthesis, MolSys Research Unit, University of Liège, B-4000 Liège (Sart Tilman), Belgium

Keywords

Non-symmetric carbonates; Organocatalysis; Methoxycarbonylation; Dimethyl carbonate; Hydroxycarbamates

Article Highlights

- Development of a continuous flow platform for the organocatalytic methoxycarbonylation of benzyl alcohol derivatives

- Use of dimethyl carbonate both as solvent and reagent, with potential for downstream recovery through distillation

- Selection of the most efficient organocatalyst balancing costs, selectivity and efficiency with a remarkable compatibility with diverse EDG and EWG substituents on the benzyl moiety

- Total selectivity toward the methoxycarbonylated product in all cases

- Conversion of benzyl methyl carbonates into the corresponding benzyl N-hydroxycarbamates using hydroxylamine in the presence of Gd(OTf)3 as Lewis acid activator

Abstract

An organocatalytic continuous flow process is presented for the preparation of non-symmetric benzyl carbonate derivatives. The procedure relies on dimethyl carbonate as a reagent and a solvent in conjunction with an organocatalyst for the selective methoxycarbonylation of diversely substituted benzylic alcohols. The reaction is optimized with benzyl alcohol as a model substrate, which emphasizes 1,4-diazabicyclo[2.2.2]octane (DABCO), 1,5,7-triazabicyclo[4.4.0]dec-5-ene (TBD), 2,8,9-trimethyl-2,5,8,9-tetraaza-1-phosphabicyclo[3.3.3]undecane (Verkade’s base) and phosphazene base P2-Et as suitable organocatalysts. The optimized conditions are next extended to a small library of 18 structurally diverse benzylic alcohol derivatives. High conversion (66-96%) and total selectivity (>99%) are achieved under mild conditions and short residence time, which converge toward a versatile and robust continuous flow protocol. Then, we illustrate that a set of representative benzyl methyl carbonates can be considered as stable, low toxicity surrogates of the corresponding chloroformates for the preparation of valuable benzyl N-hydroxycarbamates.

Introduction

Symmetric and non-symmetric organic carbonates occupy an important place in chemistry as precursors to a wide diversity of chemical functions and value-added compounds [1–7]. Depending on their alkyl or aryl moieties, they can serve either as alkylating [8–13] or acylating agents [14,15] for functionalization or protection purposes simply by tuning the process conditions. One of the main application areas for carbonates concerns the field of polymers where applications such as suturing or drug delivery rely on their biodegradable and innocuous properties. Carbonates are also used in the lubricants industry [6], are common precursors toward phytosanitary products such as insect repellents [16] and receive an increasing attention as electrolyte carriers for Li-based batteries [17,18]. A plethora of methods have been reported for synthesizing non-symmetrical carbonates either relying on homogeneous or heterogeneous metal-based catalysts (such as Ag, Fe, Al, V or La-based catalysts) [19–28] or organic metal-free catalysts [29,30] on a variety of substrates such as alcohols [31], epoxides [32], propargylic alcohol derivatives [33], and olefins [34] to name a few. All of these substrates can be converted to the corresponding carbonates using various carbonating agents such as phosgene (and its derivatives), CO/CO2 or dialkyl carbonates (Figure 1) [35]. Over the 2 past decades, phosgene and its derivatives such as chloroformates or 1,1 carbonyldiimidazole (CDI) were slowly put aside because of their inherent toxicity [36–41], hence leading to a thrust toward low toxicity and widely available alternative carbonation reagents such as CO2 and dialkyl carbonates. Despite the huge potential behind using CO2 as carbonation reagent, its thermodynamic stability still restricts its use to niche applications with specific catalytic systems [31,33,34,42–58]. Among dialkyl carbonates, dimethyl carbonate (DMC) has been concerned with a rapidly increasing interest as a widely available, low toxicity and mildly enthalphy-actived carbonation reagent [59–75]. Though the preparation of carbonates with DMC can be carried out in the absence of a catalyst under harsh conditions [62], various organocatalysts were reported for enabling a fine tuning of the selectivity as well as the extension of the scope toward more fragile substrates under milder conditions. Representative examples include diazabicyclo(5.4.0)undec-7-ene (DBU) by De Souza [75], tetraethylammonium pipecolinate by Baxendale [63], N-heterocyclic carbenes by Sundén [76], 2-tert-butyl-1,1,3,3-tetramethylguanidine (Barton’s base) [65,77] and tetrabutylammonium bromide by Monbaliu [64]. The seminal works of Selva and Meier particularly stand out [30,60]. For instance, Meier reported a simple method relying on 1,5,7-triazabicyclo[4.4.0]dec-5-ene (TBD) as organocatalyst [60].  In their paper, the authors rely on a very low catalyst loading (1 mol%) and mild process conditions in batch (typ. 1 h at 80 °C), which produces the coveted carbonate in good to excellent yield and selectivity with a rather diverse scope of substrates including benzyl alcohol. In a series of papers, Jain et al. likewise emphasized other organocatalysts including widely available tBuOK (20 mol%) for the carbonation of (cyclo)alkyl and benzyl alcohols under mild batch conditions (room temperature for 10 to 30 min) [69]. This process provides non-symmetric carbonates in good to excellent yields with total selectivity except for a few examples.

Diversely substituted benzyl methyl carbonates are per se useful building blocks for diverse applications in synthetic organic chemistry [78–82] and can be considered as stable and low toxicity surrogates of chloroformates. As part of a research program aiming at the preparation of a library of benzyl N-hydroxycarbamate derivatives, we became interested in studying and preparing non-symmetric benzyl methyl carbonates as key intermediates and surrogates of the more common chloroformates. Unlike their much more common hydroxamic acid analogs [83–86], the preparation of N-hydroxycarbamates is not widely documented and typically relies on toxic and unstable chloroformates, though some alternatives relying on less reactive and safer carbonates and pyrocarbonates were reported as well [87]. Benzyl N-hydroxycarbamate derivatives are versatile building blocks for the preparation of oxazine heterocycles [87–90] and other valuable compounds [91,92]. Despite major advances for the preparation of non-symmetric carbonates with DMC, remarkably, the literature still lacks protocols for the preparation of a library of structurally diverse benzyl methyl carbonates. In particular, the o-nitrobenzyl and the nitropiperonyl methyl carbonates that are easily accessible with total selectivity through this methodology are seen as novel opportunities for the preparation of photocleavable sources of acylnitroso reagents [93,94].

Figure 1. Carbonation of alcohol derivatives. a. Overview of the carbonation processes of alcohol derivatives. b. Selva’s procedure toward symmetric dibenzyl carbonate. c. Meier’s protocol toward non-symmetric carbonates under batch conditions. d. This work provides a versatile and robust continuous flow process for the preparation of valuable non-symmetric benzyl methyl carbonates.

Building upon our previous work on the development of low environmental footprint flow processes for accessing value-added chemicals with DMC as a sustainable and low toxicity carbonation reagent, a solution was sought for the preparation of non-symmetric benzyl methyl carbonates as surrogates of chloroformates. Inspired by Selva’s and Meier’s work on organocatalytic carbonation protocols, the approach presented herein relies on a careful selection of a suitable organocatalyst balancing costs, selectivity and efficiency. Furthermore, the presented approach also relies on a straightforward continuous flow process based on DMC both as reagent and solvent. Among the 14 different organocatalysts tested, 1,4-diazabicyclo[2.2.2]octane (DABCO), 1,5,7-triazabicyclo[4.4.0]dec-5-ene (TBD), 2,8,9-trimethyl-2,5,8,9-tetraaza-1-phosphabicyclo[3.3.3]undecane (Verkade’s base) and phosphazene base P2-Et afforded both high selectivity and conversion with a timeframe compatible with flow conditions (10 min of residence time) under mild conditions (80 °C, 100 psi of counter pressure). The combination of an efficient organocatalyst as well as the use of a large excess of DMC under flow conditions afforded very favorable conditions to shift the equilibrium toward high conversion and selectivity, while the recycling of the excess DMC through downstream batch distillation maintains a favorable environmental footprint. A library of 18 non-symmetric benzyl methyl carbonates was obtained accordingly, featuring a remarkable compatibility with diverse EDG and EWG substituents on the benzyl moiety, hence providing a convenient and versatile flow protocol towards benzyl methyl carbonates. Upon optimization, a small yet representative set of benzyl methyl carbonates was converted into the corresponding benzyl N-hydroxycarbamates using hydroxylamine in the presence of Gd(OTf)3 as catalyst.

Results and Discussion

This investigation started with a preliminary set of experiments in batch to identify a suitable organocatalyst for the methoxycarbonylation of benzyl alcohol (1a) as a model substrate with DMC. A preliminary set of organocatalysts was considered including 1,8-diazabicyclo[5.4.0]undec-7-ene (DBU), 1,4-diazabicyclo[2.2.2]octane (DABCO) and sodium methoxide to assess the boundaries of the reaction of 1a and DMC. DABCO and DBU are known to activate carbonates and are often considered as reference catalysts for the methoxycarbonylation of various substrates [74,95–97]. Preliminary experiments relied on the use of 3 mol% of catalysts as well as DMC being used as both solvent and reagent (66 equivalents). The reaction was performed in refluxing DMC (90 °C) overnight (12 h). Regarding the potential side products, 3 additional compounds are expected besides the desired benzyl methyl carbonate (2a), namely, dibenzyl carbonate (3), benzyl methyl ether (4) and dibenzyl ether (5). While dibenzyl carbonate (3) is expected to become a competitive side-product with a lower excess of DMC [12], the formation of compounds 4 and 5 is typically observed at higher process temperatures [8–13]. Commercial samples of compounds 2-5 were purchased and used as analytical references. DABCO and sodium methoxide gave after 12 h a conversion of 98% and 96%, respectively, and a high selectivity (>99%), though dibenzylcarbonate (3) was detected in the experiment with NaOMe. DBU gave a less favorable conversion (80%), yet a total selectivity was maintained. It is however interesting to point out that benzyl methyl ether (4) and dibenzyl ether (5) were not detected in these trials. Based on these preliminary results, DABCO was selected for further optimization of the reaction parameters including the excess DMC, the reaction time and the catalyst loading. The results clearly emphasized that a larger excess in DMC had a positive impact on both the conversion and selectivity, while a higher loading of DABCO had a deleterious effect on the conversion. Selectivity remained well above 95% in all cases and dibenzyl carbonate (3) was the only byproduct observed with DABCO as a catalyst.

However, the timeframe to access high conversion still seemed out of reach for a flow application. Especially since the usual temperature leverage could not be activated here to speed up the process given that high temperatures would inexorably increase the likelihood of decarboxylation and methylation competitive processes, leading to compounds 4 and 5. Therefore, the screening of catalysts was extended to a total of 14 different organocatalysts (Figure 2). The temperature had to be decreased down to 80 °C to prevent the deactivation of some organocatalysts and the reaction output was analyzed after 2 h, though maximal conversion was already reached for some organocatalysts after 10 min. Among the 14 different organocatalysts tested, 1,4-diazabicyclo[2.2.2]octane (DABCO), 1,5,7-triazabicyclo[4.4.0]dec-5-ene (TBD), 2,8,9-trimethyl-2,5,8,9-tetraaza-1-phosphabicyclo[3.3.3]undecane (Verkade’s base) and phosphazene base P2-Et afforded both high conversion and selectivity within a timeframe compatible with a flow application. Based on a subtle balance taking into account costs and efficiency, 1,5,7-triazabicyclo[4.4.0]dec-5-ene (TBD) emerged as the best compromise for further optimization in flow.

Figure 2. Catalyst screening for the dimethyl carbonate-mediated carbonation of benzyl alcohol 1a towards model benzyl methyl carbonate 2a under batch conditions. Typical condition: DMC = 66 equiv., 1a = 1 equiv., catalyst = 3 mol%, 80 °C, 2 h. Conversion and selectivity were determined by GC/FID.

A simple exploratory microfluidic setup was constructed from PFA coils and PEEK connectors to enable the fast screening of flow conditions while using minimal amounts of chemicals. PFA coils with various internal volumes were utilized when appropriate for assessing the effect of the residence time on the reaction. In a typical experiment, two distinct liquid feeds were prepared: one containing model substrate 1a dissolved in DMC (0.36 M) and the second one containing the organocatalyst TBD (0.011 M, 3 mol%) dissolved in DMC. The organocatalyst was segregated from substrate 1a to avoid any reaction upon standing in the feed solutions. Both liquid feeds were mixed through a PEEK static T-mixer and reacted in a thermostatized PFA coil under 100 psi of counter pressure (Table 1) and the conditions were reoptimized to reach the best conversion in flow. The best conversion (96%) with a selectivity >99% was obtained within 10 min of residence time at 80 °C with 3 mol% (Table 1, entry 6). Increasing the loading of catalyst or the residence time did not improve the conversion.

Table 1. Results of the optimization of the continuous flow carbonation towards model benzyl methyl carbonate 2a.

| Entry | TBD (mol%) | Time (min) | T (°C) | Conversion (%)a |
| --- | --- | --- | --- | --- |
| 1 | 1 | 10 | 40 | 24 |
| 2 | 1 | 10 | 60 | 31 |
| 3 | 1 | 10 | 80 | 56 |
| 4 | 3 | 10 | 40 | 42 |
| 5 | 3 | 10 | 60 | 55 |
| 6 | 3 | 10 | 80 | 96 |
| 7 | 5 | 10 | 40 | 56 |
| 8 | 5 | 10 | 60 | 85 |
| 9 | 5 | 10 | 80 | 89 |
| 10 | 10 | 10 | 40 | 84 |
| 11 | 10 | 10 | 60 | 93 |
| 12 | 10 | 10 | 80 | 93 |
| 13 | 3 | 1 | 80 | 24 |
| 14 | 3 | 5 | 80 | 76 |
| 15 | 3 | 15 | 80 | 94 |

Typical condition: DMC = 66 equiv., 1a = 1 equiv., TBD = 3 mol%, 100 psi. Selectivity was >99% in all cases. aConversion was determined by GC/FID.

With the optimized conditions in hands, the robustness and the scope of the process were assessed transposing the conditions to a library of benzyl alcohol derivatives featuring diverse electron withdrawing groups (EWG) or electron donating groups (EDG). The method withstands a large variety of benzyl alcohol derivatives bearing either EDG or EWG moieties with conversions ranging from 66 to 93% and selectivity >99% in all cases, which enable to devise a simple yet efficient flow system for the elaboration of a library of non-symmetric benzyl methyl carbonate derivatives 2a-r simply by changing the nature of the feed containing substrate 1a-r and operating the same reactor setup under 80 °C (100 psi of counter pressure).  Regarding the electronic effect of the substituents on the overall reactivity, there are no clear-cut trends that emerged. EWG bearing derivatives lead to a slightly lower conversion compared to their EDG counterparts regardless of their relative position on the aromatic ring (Figure 3).

Figure 3. Scope of the carbonation of benzyl alcohol derivatives under microfluidic conditions; DMC = 66 equiv., 1a-r = 1 equiv., TBD = 3 mol%, 10 min of residence time, 80 °C, 100 psi. The conversion and isolated yield (in parentheses) are indicated. Conversions were determined by GC/FID. aConversion was determined by HPLC/DAD (processed at 355 nm).

With now a convenient and straightforward access to a library of structurally diverse benzyl methyl carbonates 2, their use for the preparation of valuable benzyl N-hydroxycarbamates using hydroxylamine in the presence of Gd(OTf)3 as Lewis activator (0.5 equiv.) was envisioned. The addition of a nucleophile onto a non-symmetric carbonate raises concerns regarding the selectivity of the addition, which was discussed by Selva and others [98]. The competition between the elimination of the benzyloxy moiety vs the elimination of the methoxy group gives at best a 50% selectivity toward the desired benzyl N-hydroxycarbamates, though the competitive elimination leading to the release of the benzyloxy moiety can be adapted for recycling benzyl alcohol derivatives 1. To showcase the applicability and interest of our methodology, photocleavable benzyl N-hydroxycarbamates 6o,p were prepared under continuous flow conditions from their methoxycarbonate precursors. Benzyl hydroxycarbamate 6a, which is commercially available (CAS 3426-71-9) and used a classical Cbz-protected source of reactive nitrosocarbonyl derivatives [89,99–101], was prepared likewise as a comparison. It is noteworthy that the preparation of derivatives 6o,p according to the most common protocol relies on the corresponding chloroformates, the preparation and handling of which are cumbersome due to their high moisture sensibility [102,103]. Starting from compounds 2a and 2o,p obtained from the TBD-catalyzed flow process described above, a simple flow system was devised to enable their conversion into benzyl N-hydroxycarbamates 6a and 6o,p (Figure 4).

Figure 4. Optimized hydroxylamination process towards the preparation of a small scope of valuable hydroxycarbamate synthons under continuous flow conditions; 2 = 1 equiv., Gd(OTf)3 = 0.5 equiv., NH2OH = 5 equiv., 30 min of residence time, 80 °C, 130 psi. For the full optimization details see Supporting Information sections 2.2.5 – 2.2.6. The conversion and selectivity (in parentheses) are indicated. Conversion and selectivity were determined by HPLC/DAD processed at (6a = 193 nm, 6o = 355 nm, 6p = 267 nm).

Conclusion

A simple organocatalyzed flow process is reported for the selective methoxycarbonylation of a library of benzyl alcohol derivatives. The process relies on dimethyl carbonate both as solvent and reagents and uses TBD (3 mol%) as a homogeneous organocatalyst, providing unprecedented selectivities toward the desired benzyl methyl carbonate derivatives. The excess dimethyl carbonate is recycled through downstream distillation. The protocol is amenable to the preparation of a library of valuable non-symmetric benzyl carbonates with a remarkable compatibility with diverse EDG and EWG substituents as potential surrogates of the corresponding chloroformates toward benzyl N-hydroxycarbamates.

Experimental section

General information

Conversion and selectivity were determined by gas chromatography coupled to a flame ionization detector (GC/FID) or by high performance liquid chromatography coupled to Diode-Array Detection (HPLC/DAD). Structures were confirmed by 1H and 13C NMR spectroscopy (400 MHz Bruker Avance spectrometer) in CDCl3 (Supporting Information). The chemical shifts are reported in ppm relative to TMS as internal standard or to solvent residual peak. Solvents were used as received, unless otherwise stated. Benzyl alcohol, 4-(trifluoromethyl)benzyl alcohol, 4-fluorobenzyl alcohol, 4-chlorobenzyl alcohol, 4-bromobenzyl alcohol, 4-iodobenzyl alcohol, 3,4-difluorobenzyl alcohol, 3-chlorobenzyl alcohol, 3,4-dichlorobenzyl alcohol, 4-methylbenzyl alcohol, 2-methoxybenzyl alcohol, 3-methoxybenzyl alcohol, 4-methoxybenzyl alcohol, 3,4-dimethoxybenzyl alcohol, (6-nitrobenzo[d][1,3]dioxol-5-yl)methanol, 2-nitrobenzyl alcohol, 3-nitrobenzyl alcohol, 4-nitrobenzyl alcohol, benzyl methyl ether, dibenzyl carbonate, dimethyl carbonate, benzyl methyl carbonate, dibenzyl ether, 1,8-diazabicyclo[5.4.0]undec-7-ene (DBU), 1,4-diazabicyclo[2.2.2]octane (DABCO), morpholine, bis(dimethylamino)naphtalene (Proton Sponge), 1,5,7-triazabicyclo[4.4.0]dec-5-ene (TBD), 1,1,3,3-tetramethylguanidine (TMG), 2-tert-butyl-1,1,3,3-tetramethylguanidine (Barton’s Base), 2,8,9-trimethyl-2,5,8,9-tetraza-1-phosphabicyclo[3.3.3]undecane (Verkade’s Base), tert-butylimino-tris(dimethylamino)phosphorane, (P1-t-Bu), tetramethyl-(tris-(dimethylamino)phosphoranylidene)phosphorictriamid-ethyl-imine (P2-Et), tert-butylimino-tri(pyrrolidino)phosphorane (BTPP), potassium tert-butoxide, sodium methoxide (25 wt-% in methanol), hydroxylamine free base (50 wt-% in water), zinc triflate, copper triflate, erbium triflate, europium triflate, gadolinium triflate, hafnium triflate, lanthanum triflate, neodymium triflate, scandium triflate and silver triflate were obtained from commercial sources and used as received.

Experimental setup

Microfluidic setup for the dimethyl carbonate-mediated carbonation of benzyl alcohol derivatives. The reactor for the dimethyl carbonate-mediated carbonation of benzyl alcohol derivatives consisted of a modular continuous flow assembly made of high purity PFA capillary coils (1.58 mm outer diameter, 750 µm internal diameter) equipped with Super Flangeless nuts and ferrules (IDEX/Upchurch Scientific). Feed and collection lines consisted of PFA tubing (1.58 mm outer diameter, 750 µm internal diameter) equipped with PEEK/ETFE connectors and ferrules (IDEX/Upchurch Scientific). High force Chemyx Fusion 6000 syringe pumps (with SS syringes and Dupont Kalrez O-rings) equipped with check valves (IDEX/Upchurch Scientific) were used for handling the liquid feeds. Thermoregulation was operated with a Heidolph MR Hei-Tec® equipped with a Pt-1000 temperature sensor. Downstream pressure was regulated with a spring-loaded back pressure regulator (100 psi, IDEX/Upchurch Scientific) embedded in a cartridge holder.

Microfluidic setup for the hydroxylamination of non-symmetrical carbonates towards benzyl N-hydroxycarbamate derivatives. The reactor for the hydroxylamination of non-symmetrical carbonate derivatives consisted of a modular continuous flow assembly made of high purity PFA capillary coils (3.17 mm outer diameter, 1.65 mm internal diameter) equipped with Super Flangeless nuts and ferrules (IDEX/Upchurch Scientific). Feed and collection lines consisted of PFA tubing (3.17 mm outer diameter, 1.65 mm internal diameter) equipped with PEEK/ETFE connectors and ferrules (IDEX/Upchurch Scientific). High force Chemyx Fusion 6000 syringe pumps (with SS syringes and Dupont Kalrez O-rings) equipped with check valves (IDEX/Upchurch Scientific) were used for handling the liquid feed. Thermoregulation was operated with a Heidolph MR Hei-Tec® equipped with a Pt-1000 temperature sensor. Downstream pressure was regulated with a Zaiput Flow Technology® dome-type back pressure regulator (BPR-10, 130 psi) connected to a nitrogen tank.

Typical runs

Continuous flow preparation of non-symmetrical carbonates (2a-r). The pumps used to deliver the solutions of benzyl alcohol derivative 1a (0.36 M in DMC, 1 equiv.) and TBD (0.011 M in DMC, 0.03 equiv.) were both set at a flow rate of 0.1 mL min-1 and the two streams were mixed through a PEEK T-mixer. The homogeneous mixture was reacted in a PFA capillary coil (2 mL internal volume, 10 min of residence time) heated at 80 °C under 100 psi of counter pressure. The reactor effluent was collected at steady state and analyzed by GC/FID.

Continuous flow preparation of benzyl N-hydroxycarbamates (6a,o,p). The solution of non-symmetrical carbonate (2, 0.5 M), Gd(OTf)3 (0.25 M) and NH2OH (2.5 M) in THF was injected at a flow rate of 0.1 mL min-1 into the PFA capillary coil reactor (3 mL internal volume, 30 min of residence time) heated at 80 °C under 130 psi of counter pressure. The reactor effluent was collected at steady state and analyzed by HPLC/DAD and LC-MS for the identification of compounds.

Acknowledgments

This work was supported by the F.R.I.A.-FNRS (Fonds pour la Formation à la Recherche dans l’Industrie et dans l’Agriculture, Belgium) and the University of Liège (Welcome Grant WG-13/03, JCMM). TT is a F.R.I.A.-FNRS PhD student fellow.

Conflict of interest

There are no conflicts to declare.

Corresponding Author

* jc.monbaliu@uliege.be (Jean-Christophe M. Monbaliu)

ORCID 0000-0001-6916-8846

References

(1) 	Curini M, Epifano F, Maltese F, Rosati O (2002) Carbamate Synthesis from Amines and Dimethyl Carbonate under Ytterbium Triflate Catalysis. Tetrahedron Lett 43:4895–4897. https://doi.org/10.1016/S0040-4039(02)00965-6.

(2) 	Endo A, Danishefsky SJ (2005) Total Synthesis of Salinosporamide A. J Am Chem Soc 127:8298–8299. https://doi.org/10.1021/JA0522783.

(3) 	Studnev YN, Frolovskii VA, Kinash OF, Stolyarov VP (2006) Synthesis and Bactericidal Activity of Poly(Fluoroalkylcarbamates). Pharm Chem J 40:76–78. https://doi.org/10.1007/s11094-006-0062-2.

(4) 	Trost BM, Czabaniuk LC (2013) Palladium-Catalyzed Asymmetric Benzylation of Azlactones. Chem - A Eur J 19:15210–15218. https://doi.org/10.1002/chem.201302390.

(5) 	Ganton MD, Kerr MA (2005) A Domino Amidation Route to Indolines and Indoles:  Rapid Syntheses of Anhydrolycorinone, Hippadine, Oxoassoanine, and Pratosine. Org Lett 7:4777–4779. https://doi.org/10.1021/OL052086C.

(6) 	Parrish JP, Salvatore RN, Jung KW (2000) Perspectives on Alkyl Carbonates in Organic Synthesis. Tetrahedron 56:8207–8237. https://doi.org/10.1016/S0040-4020(00)00671-2.

(7) 	Barcelo G, Senet J-P, Sennyey G, Bensoam J, Loffet A (1986) Alkyl 1-Chloroalkyl Carbonates: Reagents for the Synthesis of Carbamates and Protection of Amino Groups. Synthesis (Stuttg) 1986:627–632. https://doi.org/10.1055/s-1986-31724.

(8) 	Tundo P, Memoli S, Hérault D, Hill K (2004) Synthesis of Methylethers by Reaction of Alcohols with Dimethylcarbonate Green Chem 6:609–612. https://doi.org/10.1039/B412722F.

(9) 	Selva M, Militello E, Fabris M (2008) The Methylation of Benzyl-Type Alcohols with Dimethyl Carbonate in the Presence of Y- and X-Faujasites: Selective Synthesis of Methyl Ethers. Green Chem 10:73–79. https://doi.org/10.1039/B707774B.

(10) 	Tundo P, Aricò F, Rosamilia AE, Memoli S (2008) Synthesis of Dialkyl Ethers by Decarboxylation of Dialkyl Carbonates. Green Chem 10:1182-1189. https://doi.org/10.1039/b809271k.

(11) 	Selva M, Benedet V, Fabris M (2012) Selective Catalytic Etherification of Glycerol Formal and Solketal with Dialkyl Carbonates and K2CO3. Green Chem 14:188–200. https://doi.org/10.1039/C1GC15796E.

(12) 	Fiorani G, Selva M (2014) Synthesis of Dibenzyl Carbonate: Towards a Sustainable Catalytic Approach. RSC Adv 4:1929–1937. https://doi.org/10.1039/C3RA42904K.

(13) 	Hemming EB, Masters AF, Perosa A, Selva M, Maschmeyer T (2019) Single-Step Methylation of Chitosan Using Dimethyl Carbonate as a Green Methylating Agent. Molecules 24:3986. https://doi.org/10.3390/molecules24213986.

(14) 	Wang C, Fan X, Chen F, Qian P-C, Cheng J (2021) Vinylene Carbonate: Beyond the Ethyne Surrogate in Rhodium-Catalyzed Annulation with Amidines toward 4-Methylquinazolines. Chem Commun 57:3929–3932. https://doi.org/10.1039/D1CC00882J.

(15) 	Tundo P, Musolino M, Aricò F (2018) The Reactions of Dimethyl Carbonate and Its Derivatives. Green Chem 20:28–85. https://doi.org/10.1039/C7GC01764B.

(16) 	Garson LR, Garner DD (1971) Unsymmetrical Carbonates as Potential Long-Lasting Insect Repellents. J Pharm Sci 60:1083–1085. https://doi.org/10.1002/jps.2600600719.

(17) 	Köps L, Leibing C, Hess LH, Balducci A (2021) Mixtures of Glyoxylic Acetals and Organic Carbonates as Electrolytes for Lithium-Ion Batteries. J Electrochem Soc 168:010513. https://doi.org/10.1149/1945-7111/abd604.

(18) 	Kühnel R-S, Böckenfeld N, Passerini S, Winter M, Balducci A (2011) Mixtures of Ionic Liquid and Organic Carbonate as Electrolyte with Improved Safety and Performance for Rechargeable Lithium Batteries. Electrochim Acta 56:4092–4099. https://doi.org/10.1016/J.ELECTACTA.2011.01.116.

(19) 	Zhou Z-H, Song Q-W, He L-N (2017) Silver(I)-Promoted Cascade Reaction of Propargylic Alcohols, Carbon Dioxide, and Vicinal Diols: Thermodynamically Favorable Route to Cyclic Carbonates. ACS Omega 2:337–345. https://doi.org/10.1021/acsomega.6b00407.

(20) 	Chen F, Liu N, Dai B (2017) Iron(II) Bis-CNN Pincer Complex-Catalyzed Cyclic Carbonate Synthesis at Room Temperature. ACS Sustain Chem Eng 5:9065–9075. https://doi.org/10.1021/acssuschemeng.7b01990.

(21) 	Verma S, Si MK, Kureshy RI, Nazish M, Kumar M, Khan NH, Abdi SHR, Bajaj HC, Ganguly B (2016) Chemical Fixation of CO2 to Cyclic Carbonates Using Al(III) β-Aminoalcohol Based Efficient Catalysts: An Experimental and Computational Studies. J Mol Catal A Chem 417:135–144. https://doi.org/10.1016/J.MOLCATA.2016.03.018.

(22) 	Wang Q, Sun MM, Guo M, Luo SJ, Li CH, Hu CW (2015) Heterogeneous Transesterification of Dimethyl Carbonate and Phenol to Diphenyl Carbonate over Vanadium Pentoxide Microstructures. Integr Ferroelectr 164:154–164. https://doi.org/10.1080/10584587.2015.1045399.

(23) 	Hatano M, Kamiya S, Ishihara K (2012) In Situ Generated “Lanthanum(III) Nitrate Alkoxide” as a Highly Active and Nearly Neutral Transesterification. Catalyst Chem Commun 48:9465-9467. https://doi.org/10.1039/c2cc34987f.

(24) 	Ramesh S, Indukuri K, Riant O, Debecker DP (2018) Synthesis of Carbonate Esters by Carboxymethylation Using NaAlO2 as a Highly Active Heterogeneous Catalyst. Org Process Res Dev 22:1846–1851. https://doi.org/10.1021/acs.oprd.8b00333.

(25) 	Ramesh S, Devred F, Debecker DP (2019) NaAlO2 Supported on Titanium Dioxide as Solid Base Catalyst for the Carboxymethylation of Allyl Alcohol with DMC. Appl Catal A Gen 581:31–36. https://doi.org/10.1016/J.APCATA.2019.05.017.

(26) 	Hatano M, Kamiya S, Moriyama K, Ishihara K (2011) Lanthanum(III) Isopropoxide Catalyzed Chemoselective Transesterification of Dimethyl Carbonate and Methyl Carbamates. Org Lett 13:430–433. https://doi.org/10.1021/ol102754y.

(27) 	Kanakikodi KS, Churipard SR, Halgeri AB, Maradur SP (2020) Solid Acid Catalyzed Carboxymethylation of Bio-Derived Alcohols: An Efficient Process for the Synthesis of Alkyl Methyl Carbonates. Sci Rep 10:13103. https://doi.org/10.1038/s41598-020-69989-7.

(28) 	Chevella D, Macharla AK, Banothu R, Gajula KS, Amrutham V, Boosa M, Nama N (2019) Synthesis of Non-Symmetrical Alkyl Carbonates from Alcohols and DMC over the Nanocrystalline ZSM-5 Zeolite. Green Chem 21:2938–2945. https://doi.org/10.1039/C9GC00541B.

(29) 	Cokoja M, Wilhelm ME, Anthofer MH, Herrmann WA, Kühn FE (2015) Synthesis of Cyclic Carbonates from Epoxides and Carbon Dioxide by Using Organocatalysts. ChemSusChem 8:2436–2454. https://doi.org/10.1002/cssc.201500161.

(30) 	Selva M, Noè M, Perosa A, Gottardo M (2012) Carbonate, Acetate and Phenolate Phosphonium Salts as Catalysts in Transesterification Reactions for the Synthesis of Non-Symmetric Dialkyl Carbonates. Org Biomol Chem 10:6569-6578. https://doi.org/10.1039/c2ob25447f.

(31) 	Dabral S, Licht U, Rudolf P, Bollmann G, Hashmi ASK, Schaub T (2020) Synthesis and Polymerisation of α-Alkylidene Cyclic Carbonates from Carbon Dioxide, Epoxides and the Primary Propargylic Alcohol 1,4-Butynediol. Green Chem 22:1553–1558. https://doi.org/10.1039/C9GC04320A.

(32) 	Ma H, Zeng J, Tu D, Mao W, Zhao B, Wang K, Liu Z, Lu J (2020) 3-Bromo-1,1,1-Trifluoro-2-Propanol Assisted Chemical Fixation of CO2 and Epoxides. Tetrahedron Lett 61:151593. https://doi.org/10.1016/J.TETLET.2020.151593.

(33) 	Han L-H, Li J-Y, Song Q-W, Zhang K, Zhang Q-X, Sun X-F, Liu P (2020) Thermodynamic Favorable CO2 Conversion via Vicinal Diols and Propargylic Alcohols: A Metal-Free Catalytic Method. Chinese Chem Lett 31:341–344. https://doi.org/10.1016/J.CCLET.2019.06.030.

(34) 	Zalomaeva OV, Maksimchuk NV, Chibiryaev AM, Kovalenko KA, Fedin VP, Balzhinimaev BS (2013) Synthesis of Cyclic Carbonates from Epoxides or Olefins and CO2 Catalyzed by Metal-Organic Frameworks and Quaternary Ammonium Salts. J Energy Chem 22:130–135. https://doi.org/10.1016/S2095-4956(13)60017-0.

(35) 	Li Y, Wang Z, Wu X-F (2018) A Sustainable Procedure toward Alkyl Arylacetates: Palladium-Catalysed Direct Carbonylation of Benzyl Alcohols in Organic Carbonates. Green Chem 20:969–972. https://doi.org/10.1039/C7GC03619A.

(36) 	Řimnáčová L, Hušek P, Šimek P (2014) A New Method for Immediate Derivatization of Hydroxyl Groups by Fluoroalkyl Chloroformates and Its Application for the Determination of Sterols and Tocopherols in Human Serum and Amniotic Fluid by Gas Chromatography–Mass Spectrometry. J Chromatogr A 1339:154–167. https://doi.org/10.1016/J.CHROMA.2014.03.007.

(37) 	Kaiser A, Richert C (2010) Azidomethyl 4-Nitrophenyl Carbonate - A Reagent for the One-Step Introduction of the Azidomethyloxycarbonyl (Azoc) Protecting Group. Synlett 2010:2267–2270. https://doi.org/10.1055/s-0030-1258049.

(38) 	Liang F, Yanai M, Suzuki Y, Tsuda A (2020) Photo-on-Demand Synthesis of Chloroformates with a Chloroform Solution Containing an Alcohol and Its One-Pot Conversion to Carbonates and Carbamates. Org Lett 22:3566–3569. https://doi.org/10.1021/acs.orglett.0c01013.

(39) 	Wondraczek H, Elschner T, Heinze T (2011) Synthesis of Highly Functionalized Dextran Alkyl Carbonates Showing Nanosphere Formation. Carbohydr Polym 83:1112–1118. https://doi.org/10.1016/J.CARBPOL.2010.09.013.

(40) 	Elschner T, Wondraczek H, Heinze T (2013) Syntheses and Detailed Structure Characterization of Dextran Carbonates. Carbohydr Polym 93:216–223. https://doi.org/10.1016/J.CARBPOL.2012.01.091.

(41) 	Sung GH, Kim BR, Ryu KE, Kim J-J, Yoon Y-J (2015) Synthesis of Organic Carbonates with Alkyl/Aryl 4,5-Dichloro-6-Oxopyridazine-1(6H)-Carboxylates and ROH/AlCl3 under Ambient Condition. Bull Kor Chem Soc 35:2758-2764. https://doi.org/10.5012/bkcs.2014.35.9.2758.

(42) 	McGuire TM, López-Vidal EM, Gregory GL, Buchard A (2018) Synthesis of 5- to 8-Membered Cyclic Carbonates from Diols and CO2: A One-Step, Atmospheric Pressure and Ambient Temperature Procedure. J CO2 Util 27:283–288. https://doi.org/10.1016/J.JCOU.2018.08.009.

(43) 	Minakata S, Sasaki I, Ide T (2010) Atmospheric CO2 Fixation by Unsaturated Alcohols Using TBuOI under Neutral Conditions. Angew Chemie Int Ed 49:1309–1311. https://doi.org/10.1002/anie.200906352.

(44) 	Wu L-X, Wang H, Xiao Y, Tu Z-Y, Ding B-B, Lu J-X (2012) Synthesis of Dialkyl Carbonates from CO2 and Alcohols via Electrogenerated N-Heterocyclic Carbenes Electrochem commun 25:116–118. https://doi.org/10.1016/J.ELECOM.2012.09.028.

(45) 	Ca’ ND, Gabriele B, Ruffolo G, Veltri L, Zanetta T, Costa M (2011) Effective Guanidine-Catalyzed Synthesis of Carbonate and Carbamate Derivatives from Propargyl Alcohols in Supercritical Carbon Dioxide. Adv Synth Catal 353:133–146. https://doi.org/10.1002/adsc.201000607.

(46) 	Wang Y-B, Wang Y-M, Zhang W-Z, Lu X-B (2013) Fast CO2 Sequestration, Activation, and Catalytic Transformation Using N-Heterocyclic Olefins. J Am Chem Soc 135:11996–12003. https://doi.org/10.1021/ja405114e.

(47) 	Lim YN, Lee C, Jang H-Y (2014) Metal-Free Synthesis of Cyclic and Acyclic Carbonates from CO2 and Alcohols. European J Org Chem 2014:1823–1826. https://doi.org/10.1002/ejoc.201400031.

(48) 	Wang Y-B, Sun D-S, Zhou H, Zhang W-Z, Lu X-B (2014) Alkoxide-Functionalized Imidazolium Betaines for CO2 Activation and Catalytic Transformation. Green Chem 16:2266–2272. https://doi.org/10.1039/C3GC42346H.

(49) 	Qiu J, Zhao Y, Li Z, Wang H, Fan M, Wang J (2017) Efficient Ionic-Liquid-Promoted Chemical Fixation of CO2 into α-Alkylidene Cyclic Carbonates. ChemSusChem 10:1120–1127. https://doi.org/10.1002/cssc.201601129.

(50) 	Wu Y, Zhao Y, Li R, Yu B, Chen Y, Liu X, Wu C, Luo X, Liu Z (2017) Tetrabutylphosphonium-Based Ionic Liquid Catalyzed CO2 Transformation at Ambient Conditions: A Case of Synthesis of α-Alkylidene Cyclic Carbonates. ACS Catal 7:6251–6255. https://doi.org/10.1021/acscatal.7b01422.

(51) 	Goodrich P, Gunaratne NHQ, Jin L, Lei Y, Seddon KR (2017) Carbon Dioxide Utilisation for the Synthesis of Unsymmetrical Dialkyl and Cyclic Carbonates Promoted by Basic Ionic Liquids. Aust J Chem 71:181–185. https://doi.org/10.1071/CH17530.

(52) 	Khokarale SG, Mikkola J-P (2019) Metal Free Synthesis of Ethylene and Propylene Carbonate from Alkylene Halohydrin and CO2 at Room Temperature. RSC Adv 9:34023–34031. https://doi.org/10.1039/C9RA06765E.

(53) 	Yan S, Zhou R, Han F, Feng M, Miao C, Zhang S, Ai S (2020) A CO2-Induced ROCO2 Na/ROCO2H Buffer Solution Promoted the Carboxylative Cyclization of Propargyl Alcohol to Synthesize Cyclic Carbonates. Catal Sci Technol 10:736–741. https://doi.org/10.1039/C9CY02311A.

(54) 	Ochoa-Gómez JR, Gómez-Jiménez-Aberasturi O, Ramírez-López CA, Nieto-Mestre J, Maestro-Madurga B, Belsué M (2011) Synthesis of Glycerol Carbonate from 3-Chloro-1,2-Propanediol and Carbon Dioxide Using Triethylamine as Both Solvent and CO2 Fixation–Activation Agent. Chem Eng J 175:505–511. https://doi.org/10.1016/J.CEJ.2011.09.081.

(55) 	Zhou H, Wang G-X, Lu X-B (2017) CO2 Adducts of α-Carbon Alkylated N-Heterocyclic Olefins: Highly Active Organocatalysts for CO2 Chemical Transformation. Asian J Org Chem 6:1264–1269. https://doi.org/10.1002/ajoc.201700152.

(56) 	Boyaval A, Méreau R, Grignard B, Detrembleur C, Jerome C, Tassaing T (2017) Organocatalytic Coupling of CO2 with a Propargylic Alcohol: A Comprehensive Mechanistic Study. ChemSusChem 10:1241–1248. https://doi.org/10.1002/cssc.201601524.

(57) 	Gu Q, Fang J, Xu Z, Ni W, Kong K, Hou Z (2018) CO2 Promoted Synthesis of Unsymmetrical Organic Carbonate Using Switchable Agents Based on DBU and Alcohols. New J Chem 42:13054–13064. https://doi.org/10.1039/C8NJ01638K.

(58) 	Zanda N, Sobolewska A, Alza E, Kleij AW, Pericàs MA (2021) Organocatalytic and Halide-Free Synthesis of Glycerol Carbonate under Continuous Flow. ACS Sustain Chem Eng 9:4391–4397. https://doi.org/10.1021/acssuschemeng.1c01060.

(59) 	Osumah A, Magolan J, Waynant KV (2019) One-Pot Carbonyl Reduction and Carbonate Formation Using Sodium Borohydride in Dialkyl Carbonate Solvents. Tetrahedron Lett 60:151203. https://doi.org/10.1016/J.TETLET.2019.151203.

(60) 	Mutlu H, Ruiz J, Solleder SC, Meier MAR (2012) TBD Catalysis with Dimethyl Carbonate: A Fruitful and Sustainable Alliance. Green Chem 14:1728-1735. https://doi.org/10.1039/c2gc35191a.

(61) 	Kumar S, Jain SL (2013) Non-Symmetrical Dialkyl Carbonate Synthesis Promoted by 1-(3-Trimethoxysilylpropyl)-3-Methylimidazolium Chloride. New J Chem 37:3057-3061. https://doi.org/10.1039/c3nj00640a.

(62) 	Kumar S, Jain SL (2014) An Easy Base-Assisted Synthesis of Unsymmetrical Carbonates from Alcohols with Dimethyl Carbonate. Monatshefte für Chemie - Chem Mon 145:791–795. https://doi.org/10.1007/s00706-013-1123-3.

(63) 	Semenova AM, Pervova MG, Ezhikova MA, Kodess MI, Zapevalov AY, Pestov AV (2019) Transesterification of Dialkyl Carbonates with 2,2,3,3-Tetrafluoropropan-1-Ol. Russ J Org Chem 55:771–774. https://doi.org/10.1134/S1070428019060058.

(64) 	Chiappe C, Rajamani S (2012) Synthesis of Glycerol Carbonate from Glycerol and Dimethyl Carbonate in Basic Ionic Liquids. Pure Appl Chem 84:755–762. https://doi.org/10.1351/PAC-CON-11-07-06.

(65) 	Munshi MK, Gade SM, Mane MV, Mishra D, Pal S, Vanka K, Rane VH, Kelkar AA (2014) 1,8-Diazabicyclo[5.4.0]Undec-7-Ene (DBU): A Highly Efficient Catalyst in Glycerol Carbonate Synthesis. J Mol Catal A Chem 391:144–149. https://doi.org/10.1016/J.MOLCATA.2014.04.016.

(66) 	Hervert B, McCarthy PD, Palencia H (2014) Room Temperature Synthesis of Glycerol Carbonate Catalyzed by N-Heterocyclic Carbenes. Tetrahedron Lett 55:133–136. https://doi.org/10.1016/J.TETLET.2013.10.135.

(67) 	Simanjuntak FSH, Choi JS, Lee G, Lee HJ, Lee SD, Cheong M, Kim HS, Lee H (2015) Synthesis of Glycerol Carbonate from the Transesterification of Dimethyl Carbonate with Glycerol Using DABCO and DABCO-Anchored Merrifield Resin. Appl Catal B Environ 165:642–650. https://doi.org/10.1016/J.APCATB.2014.10.071.

(68) 	Nogueira DO, de Souza SP, Leão RAC, Miranda LSM, de Souza ROMA (2015) Process Intensification for Tertiary Amine Catalyzed Glycerol Carbonate Production: Translating Microwave Irradiation to a Continuous-Flow Process. RSC Adv 5:20945–20950. https://doi.org/10.1039/C5RA02117K.

(69) 	Wang X, Zhang P, Cui P, Cheng W, Zhang S (2017) Glycerol Carbonate Synthesis from Glycerol and Dimethyl Carbonate Using Guanidine Ionic Liquids. Chinese J Chem Eng 25:1182–1186. https://doi.org/10.1016/J.CJCHE.2017.06.025.

(70) 	Guidi S, Calmanti R, Noè M, Perosa A, Selva M (2016) Thermal (Catalyst-Free) Transesterification of Diols and Glycerol with Dimethyl Carbonate: A Flexible Reaction for Batch and Continuous-Flow Applications. ACS Sustain Chem Eng 4:6144–6151. https://doi.org/10.1021/acssuschemeng.6b01633.

(71) 	Van Mileghem S, De Borggraeve WM, Baxendale IR (2018) A Robust and Scalable Continuous Flow Process for Glycerol Carbonate. Chem Eng Technol 41:2014–2023. https://doi.org/10.1002/ceat.201800012.

(72) 	Gérardy R, Estager J, Luis P, Debecker DP, Monbaliu, J-CM (2019) Versatile and Scalable Synthesis of Cyclic Organic Carbonates under Organocatalytic Continuous Flow Conditions. Catal Sci Technol 9:6841–6851. https://doi.org/10.1039/C9CY01659G.

(73) 	Wang Z, Gérardy R, Gauron G, Damblon C, Monbaliu J-CM (2019) Solvent-Free Organocatalytic Preparation of Cyclic Organic Carbonates under Scalable Continuous Flow Conditions. React Chem Eng 4:17–26. https://doi.org/10.1039/C8RE00209F.

(74) 	Selva M, Perosa A, Rodríguez-Padrón D, Luque R (2019) Applications of Dimethyl Carbonate for the Chemical Upgrading of Biosourced Platform Chemicals. ACS Sustain Chem Eng 7:6471–6479. https://doi.org/10.1021/acssuschemeng.9b00464.

(75) 	Fiorani G, Perosa A, Selva M (2018) Dimethyl Carbonate: A Versatile Reagent for a Sustainable Valorization of Renewables. Green Chem 20:288–322. https://doi.org/10.1039/C7GC02118F.

(76) 	Axelsson A, Antoine-Michard A, Sundén H (2017) Organocatalytic Valorisation of Glycerol via a Dual NHC-Catalysed Telescoped Reaction. Green Chem 19:2477–2481. https://doi.org/10.1039/C7GC00471K.

(77) 	Tshibalonza NN, Gérardy R, Alsafra Z, Eppe G, Monbaliu J-CM (2018) A Versatile Biobased Continuous Flow Strategy for the Production of 3-Butene-1,2-Diol and Vinyl Ethylene Carbonate from Erythritol. Green Chem 20:5147–5157. https://doi.org/10.1039/C8GC02468E.

(78) 	Kuwano R, Kusano H (2008) Benzyl Protection of Phenols under Neutral Conditions: Palladium-Catalyzed Benzylations of Phenols. Org Lett 10:1979–1982. https://doi.org/10.1021/ol800548t.

(79) 	Zhu Y, Rawal VH (2012) Palladium-Catalyzed C3-Benzylation of Indoles. J Am Chem Soc 134:111–114. https://doi.org/10.1021/ja2095393.

(80) 	Makida Y, Usui K, Ueno S, Kuwano R (2017) Palladium-Catalyzed Benzylic Substitution of Benzyl Carbonates with Phosphorus Nucleophiles. Chem Lett 46:1814–1817. https://doi.org/10.1246/cl.170901.

(81) 	Ohsumi M, Nishiwaki N (2017) Selective Synthesis of (Benzyl)Biphenyls by Successive Suzuki–Miyaura Coupling of Phenylboronic Acids with 4-Bromobenzyl Acetate under Air Atmosphere. ACS Omega 2:7767–7771. https://doi.org/10.1021/acsomega.7b01450.

(82) 	Ohsumi M, Ito A, Nishiwaki N (2018) Substrate Switchable Suzuki–Miyaura Coupling for Benzyl Ester vs Benzyl Halide. RSC Adv 8:35056–35061. https://doi.org/10.1039/C8RA07841F.

(83) 	Alam MA (2019) Methods for Hydroxamic Acid Synthesis. Curr Org Chem 23:978–993. https://doi.org/10.2174/1385272823666190424142821.

(84) 	Sugimoto T, Kaiser ET (1978) Reaction of a Mixed Anhydride with Aqueous Hydroxylamine. A Model for the Trapping by Added Nucleophiles of Anhydride Intermediates in Carboxypeptidase A Action. J Org Chem 43:3311–3313. https://doi.org/10.1021/jo00411a010.

(85) 	Reddy AS, Kumar MS, Reddy GR (2000) A Convenient Method for the Preparation of Hydroxamic Acids. Tetrahedron Lett 41:6285–6288. https://doi.org/10.1016/S0040-4039(00)01058-3.

(86) 	Usachova N, Leitis G, Jirgensons A, Kalvinsh I (2010) Synthesis of Hydroxamic Acids by Activation of Carboxylic Acids with N,N′-Carbonyldiimidazole: Exploring the Efficiency of the Method. Synth Commun 40:927–935. https://doi.org/10.1080/00397910903026723.

(87) 	Chaiyaveij D, Batsanov AS, Fox MA, Marder TB, Whiting A (2015) An Experimental and Computational Approach to Understanding the Reactions of Acyl Nitroso Compounds in [4 + 2] Cycloadditions. J Org Chem 80:9518–9534. https://doi.org/10.1021/acs.joc.5b01470.

(88) 	Teo YC, Pan Y, Tan CH (2013) Organic Dye-Photocatalyzed Acylnitroso Ene Reaction. ChemCatChem 5:235–240. https://doi.org/10.1002/cctc.201200435.

(89) 	Monbaliu J-CM, Tinant B, Marchand-Brynaert J (2010) [4 + 2] Cycloadditions of 1-Phosphono-1,3-Butadienes with Nitroso Heterodienophiles: A Versatile Synthetic Route for Polyfunctionalized Aminophosphonic Derivatives. J Org Chem 75:5478–5486. https://doi.org/10.1021/jo100230r.

(90) 	Memeo MG, Quadrelli P (2017) Generation and Trapping of Nitrosocarbonyl Intermediates. Chem Rev 117:2108–2200. https://doi.org/10.1021/acs.chemrev.6b00684.

(91) 	Lewis TA, Bayless L, DiPesa AJ, Eckman JB, Gillard M, Libertine L, Scannell RT, Wypij DM, Young MA (2005) 5-Lipoxygenase Inhibition by N-Hydroxycarbamates in Dual-Function Compounds. Bioorg Med Chem Lett 15:1083–1085. https://doi.org/10.1016/J.BMCL.2004.12.023.

(92) 	Dickman DA, Ding H, Li Q, Nilius AM, Balli DJ, Ballaron SJ, Trevillyan JM, Smith ML, Seif LS, Kim K, Sarthy A, Goldman RC, Plattner JJ, Bennani YL (2000) Antifungal Rapamycin Analogues with Reduced Immunosuppressive Activity. Bioorg Med Chem Lett 10:1405–1408. https://doi.org/10.1016/S0960-894X(00)00184-0.

(93) 	Klán P, Šolomek T, Bochet CG, Blanc A, Givens R, Rubina M, Popik V, Kostikov A, Wirz J (2013) Photoremovable Protecting Groups in Chemistry and Biology: Reaction Mechanisms and Efficacy. Chem Rev 113:119–191. https://doi.org/10.1021/cr300177k.

(94) 	Bochet CG, Blanc A (2009) Photolabile Protecting Groups in Organic Synthesis. In Handbook of Synthetic Photochemistry; Wiley-VCH Verlag GmbH & Co KGaA: Weinheim, Germany; pp 417–447. https://doi.org/10.1002/9783527628193.ch13.

(95) 	Carafa M, Mesto E, Quaranta E (2011) DBU-Promoted Nucleophilic Activation of Carbonic Acid Diesters. European J Org Chem 2011:2458–2465. https://doi.org/10.1002/ejoc.201001725.

(96) 	Aricò F, Evaristo S, Tundo P (2015) Synthesis of Five- and Six-Membered Heterocycles by Dimethyl Carbonate with Catalytic Amounts of Nitrogen Bicyclic Bases. Green Chem 17:1176–1185. https://doi.org/10.1039/C4GC01822B.

(97) 	Selva M, Perosa A, Guidi S, Cattelan L (2016) Ionic Liquids as Transesterification Catalysts: Applications for the Synthesis of Linear and Cyclic Organic Carbonates. Beilstein J Org Chem 12:1911–1924. https://doi.org/10.3762/bjoc.12.181.

(98) 	Selva M, Trotta F, Tundo P (1992) Esters and Orthoesters as Alkylating Agents at High Temperature. Applications to Continuous-Flow Processes. J Chem Soc Perkin Trans 2 1992:519-522. https://doi.org/10.1039/p29920000519.

(99) 	Arribas C, Carmen  Carreño M, García-Ruano JL, Rodríguez JF, Santos M, Ascensión Sanz-Tejedor M (2000) First Asymmetric Hetero Diels−Alder Reaction of 1-Sulfinyl Dienes with Nitroso Derivatives. A New Entry to the Synthesis of Optically Pure 1,4-Imino-l-Ribitol Derivatives Org Lett 2:3165–3168. https://doi.org/10.1021/OL0063611.

(100) 	Zhou B, Du J, Yang Y, Feng H, Li Y (2014) Rh(III)-Catalyzed C–H Amidation with N-Hydroxycarbamates: A New Entry to N-Carbamate-Protected Arylamines. Org Lett 16:592–595. https://doi.org/10.1021/ol403477w.

(101) 	Howard JAK, Ilyashenko G, Sparkes HA, Whiting A (2007) Development of New Transition Metal Catalysts for the Oxidation of a Hydroxamic Acid with in Situ Diels–Alder Trapping of the Acyl Nitroso Derivative. Dalt Trans 21:2108–2111. https://doi.org/10.1039/B704728B.

(102) 	Negri GE, Deming TJ (2016) Triggered Copolypeptide Hydrogel Degradation Using Photolabile Lysine Protecting Groups. ACS Macro Lett 5:1253–1256. https://doi.org/10.1021/acsmacrolett.6b00715.

(103) 	Dyer RG, Turnbull KD (1999) Hydrolytic Stabilization of Protected P-Hydroxybenzyl Halides Designed as Latent Quinone Methide Precursors. J Org Chem 64:7988–7995. https://doi.org/10.1021/JO991085T.

Graphical abstract