COUPLING OF VAC-HS-SPME AND GC×GC-QMS FOR SIMULTANEOUS 5-HMF QUANTIFICATION AND VOLATILE PROFILING IN HONEY

D. Eggermont1, F. Pardi1, 2, G. Purcaro1.

(1) Gembloux Agro-Bio Tech, University of Liège, Passage des Déportés, 2, Gembloux, B-5030, Belgium., d.eggermont@uliege.be.

(2) La Sapienza, University of Roma, Piazzale Aldo Moro 5, 00185, Roma, Italy

Max 250 words!

Faced with the growing complexity of the food industry's demands, analytical techniques are evolving towards increasingly sophisticated and multi-response analyses. In this respect, multidimensional techniques such as GC-MS and GC×GC-MS are ideally suited.

Regarding sample preparation, headspace-solid-phase microextraction (HS-SPME) is one of the most favoured techniques for food volatile analysis thanks to its simplicity and ability to concentrate a wide range of compounds without needing solvents. However, the extraction of low-volatile compounds can be limited. Among different available strategies to increase their extraction, vacuum-assisted (Vac)-HS-SPME is highly promising. The use of vacuum facilitates the volatilisation of less-volatile compounds by lowering the gas-phase resistance to the mass transfer while maintaining the same extraction efficiency for the more volatile compounds.

In this project, Vac-HS-SPME has been coupled with GC×GC-qMS to quantify the 5-HMF (storage and heat processing marker) and analyse the volatile profile of honey. 5-HMF is regulated by the EU (2001/110/EC) fixing the maximum limit level to 40 mg/kg in most cases. Validation of the proposed method has been realized using a matrix-matched calibration, reaching LOD and LOQ of 1.6 and 4.7 mg/kg, respectively; while a recovery of 98% and a RDS of 21% were achieved. The method was trailed with eight real-world samples against the official HPLC method showing, an average bias of 6%. In terms of greenness the proposed method gave better results using the AGREE metrics, while the practicality was similar, as calculated with the BAGI.