# Supplementary Material

A regime-based framework for linking runoff initiation and event-scale sediment export in a small temperate agricultural loess catchment.

This Supplementary Material provides the methodological details, event-level data, parameter values, model formulations, and sensitivity results supporting the analyses presented in the main manuscript. Sections SM1 and SM2 document the monitoring system and dataset construction. Sections SM3 and SM4 describe the benchmark runoff formulations and the parameterisation of the hydrological and erosion factors. Section SM5 reports the event-level observations and diagnostic initial abstraction estimates. Section SM6 provides the complete formulation and evaluation of the continuous generalisation models, and Section SM7 presents the additional antecedent-rainfall sensitivity analysis.

# SM1. Monitoring setup and data acquisition

## This section summarises the available technical information on the acquisition of rainfall, runoff, and suspended-sediment observations. Runoff data were supplied to the authors as results previously processed by the laboratory according to data quality. Sediment data corresponded to the final results supplied by the laboratory. The original field records, monitoring logbooks, raw instrumental data, and detailed laboratory quality-control procedures were not available to the authors. Tables SM1 and SM2 summarise the processed data streams and the principal specifications of the monitoring equipment, while Figure SM1 illustrates the technical configuration of the outlet station.

## SM1.1 Gauging-flume geometry and stage-discharge conversion

The installation drawing indicates an overall flume length of approximately 1.50 m, an upstream width of approximately 0.90 m, and a wall height of approximately 0.645 m. For the installed 0.30 m throat width, the metric stage-discharge relationship used to process the water-level record was:

Q=1.932*h1.3219

where Q is discharge (L.s⁻¹) and h is water depth (cm). According to the available technical documentation, this relationship was used to convert water level into discharge. The authors received the resulting laboratory-processed discharge data and did not have access to the original station logbook or to the detailed identification of invalid monitoring intervals.

## SM1.2 Gravimetric suspended-sediment concentration

Suspended-sediment concentrations were supplied to the authors as final laboratory results. According to the available methodological information, concentration was determined gravimetrically from dry sediment mass and sampled water volume:

ci=miVi

where ci is concentration (g.L⁻¹), mi is dry sediment mass (g), and Vi is sampled water volume (L). According to the available monitoring documentation, the nominal sampled volume was 0.5 L.

## SM1.3 Structure of the laboratory-supplied dataset

The authors received a processed results dataset supplied by the laboratory. The available event-scale rainfall, runoff, discharge, and sediment variables were used without modification of the original measurements by the authors. The original continuous monitoring archive was not available to the authors. Table SM1 reports only the information required to reproduce the temporal synchronization.

Table SM1: Available information on the monitoring data streams and their nominal temporal resolution.

| Data stream | Reported variables | Stored resolution | Processing reported in the available documentation |
| --- | --- | --- | --- |
| Rainfall | Weighing- and tipping-bucket precipitation | 1 min accumulations | Aggregated to 5 min |
| Runoff | Water depth and discharge | 5 min means | Used at 5 min resolution |
| Suspended sediment | Bottle concentration and sampling time | One sample every 5 min after activation | Matched to the corresponding 5 min discharge interval |

## SM1.4 Status and limitations of the supplied dataset

## Runoff data were supplied to the authors after processing by the laboratory according to the observed data quality. Sediment data corresponded to the final laboratory results. The authors did not alter the original measurements and did not have access to the original field records, monitoring logbooks, raw instrumental series, or detailed laboratory quality-control procedures. Consequently, the precise treatment of individual missing or unreliable observations could not be reconstructed. The analyses were performed using the processed results supplied by the laboratory.

## SM1.5 Technical specifications

Table SM2: Principal technical characteristics of the monitoring equipment. Source ULiège.

| Variable or function | Instrument | Range or resolution | Reported accuracy | Role |
| --- | --- | --- | --- | --- |
| Primary rainfall | OTT Pluvio² L weighing gauge | 12-1800 mm h⁻¹; 1 min record | ±0.05 mm | Primary precipitation series |
| Rainfall control | Campbell Scientific ARG100 | 0.20 mm per tip | Not specified | Control and documented replacement |
| Water level | OTT PLS pressure probe | 0-4 m | Not specified in supplied documentation | Stage measurement |
| Logging | Sigma 950 | Level 0.005-3.5 m | Level ±0.007 m and ±2% | Acquisition of runoff-related variables |
| Gauging structure | San Dimas-type stainless-steel flume | 0.30 m throat width | Site-specific rating relation | Discharge calculation |
| Sediment sampling | Automatic 24-bottle sampler | 500 mL every 5 min; trigger h ≥ 2.5 cm | Not applicable | Event-triggered sampling |

Figure SM1: Technical configuration of the BV1p2 outlet station, including the San Dimas-type flume, its principal dimensions, the stage-discharge relationship, and the monitoring equipment. (Source: ULiège site technical documentation).

# SM2. Event selection and dataset construction

This section describes the construction of the analytical datasets from the processed results supplied by the laboratory. It presents the event-scale variables used in the study and the model-based feasibility screening subsequently performed by the authors. The laboratory processing and field-level quality-control procedures could not be reconstructed because the original monitoring records and detailed processing documentation were not available to the authors. It describes the calculation of rainfall depth, runoff volume, runoff depth, peak discharge, suspended-sediment yield, rainfall kinetic energy, and maximum 30-min rainfall intensity. It also explains the dataset-status coding and the quality-control criteria used to distinguish events included in the hydrological dataset from those with sufficiently complete sediment observations. Tables SM3 and SM3b document the complete transition from the 169 candidate rainfall–runoff events to the 154-event hydrological dataset. Table SM3 reports the retained events, whereas Table SM3b identifies the 15 events excluded because no feasible non-negative value of λcould reproduce their observed runoff under the adopted SCS–CN parameterisation.

SM2.1 SCS–CN feasibility screening

A total of 169 rainfall events generated measurable runoff and had sufficiently complete rainfall and discharge records for initial consideration. Before event-specific optimisation of the initial abstraction ratio, each event was assessed against the feasible domain of the adopted SCS–CN parameterisation.

For fixed rainfall depth Pi and potential maximum retention Si, simulated runoff decreases monotonically as the non-negative initial abstraction ratio λ increases. The maximum runoff depth representable by the model is therefore obtained at λ=0:

Rmax⁡,i=Ri(λ=0)=Pi2Pi+Si

Events satisfying  Robs,i>Rmax⁡,i had no feasible non-negative value of λ capable of reproducing the observed runoff depth. Fifteen of the 169 candidate events met this criterion and were excluded before optimisation. The remaining 154 events formed the hydrological dataset.

These inconsistencies may reflect uncertainty in rainfall or discharge measurements, limitations in the assigned annual Curve Number, or runoff responses outside the representational capacity of the adopted SCS–CN parameterisation. The available observations did not allow these possible explanations to be distinguished for each event.

## SM2.2 Event-specific optimisation of the initial abstraction ratio

For each retained event, the optimal initial abstraction ratio was estimated as:

λopt,i=arg⁡min⁡λ∈[0,λmax⁡,i]Rsim,i(λ)-Robs,i2,

where

λmax⁡,i=min⁡1,PiSi-10-12.

This upper bound ensured that Ia=λ*S<P, while the lower bound constrained λ to non-negative values. Optimisation was performed using a deterministic three-stage multi-resolution grid search. At each stage, 2,001 equally spaced candidate values were evaluated, and the interval surrounding the best-performing value was retained for the next refinement stage. Because the procedure consisted of three predefined refinement stages rather than an iterative convergence algorithm, no separate convergence tolerance was applied. Solutions located at either parameter bound were retained as valid boundary solutions.

## SM2.3 Event-scale hydrological variables

Event rainfall depth was obtained by summing the 5-min rainfall increments assigned to the event. Runoff volume was calculated by numerical integration of the 5-min discharge series:

Q=i(qi*∆t)

where Q is runoff volume (L), qi is discharge (L.s-1) during interval i, and ∆t is 300 s. Runoff depth was calculated by dividing runoff volume by the 83.84 ha catchment area. Peak discharge was the maximum 5-min discharge value within the event hydrograph.

## SM2.4 Observed event sediment yield

Each concentration measurement was assigned to the corresponding 5 min discharge interval. Observed event sediment yield SYobs was then obtained by integration over the monitored hydrograph:

SYobs=i(ci*qi*∆t)

where ci is the sediment concentration (g.L⁻¹). The resulting mass was converted from grams to tonnes. Only events for which the laboratory-supplied dataset contained a usable sediment-yield result were included in the sediment subset. The individual reasons for unavailable sediment results could not be reconstructed from the information available to the authors.

## SM2.5 Rainfall-intensity descriptors

The RUSLE2 intensity–energy relationship was selected for three reasons. First, its bounded exponential form provides a finite asymptote at high rainfall intensities, unlike an unbounded extrapolation of the original USLE relationship. Second, previous work in Belgium, including Bollinne et al. (1984), showed that rainfall erosivity relationships require consideration of the regional rainfall regime and questioned the direct application of the original relationship under Belgian temperate conditions. Third, this choice maintains methodological consistency with the RUSLE framework used for official regional erosion-sensitivity assessment in Wallonia. Bollinne et al. (1984) therefore provided regional support for the methodological choice but were not the source of the equation itself, which follows the formulation adopted in RUSLE2 (McGregor et al., 1995).

Maximum 30 min rainfall intensity (I₃₀) was defined as the largest mean intensity over any six consecutive 5 min rainfall intervals.

For each 5 min interval, unit rainfall kinetic energy was calculated using the exponential form applied in the analysis:

ei=0.29*1-0.72*exp(-0.082*Ii

where ei is unit rainfall kinetic energy (MJ.ha⁻¹.mm⁻¹) and Ii is rainfall intensity (mm.h⁻¹). Total event rainfall kinetic energy was calculated as:

E=i(ei*Pi)

where Pi is rainfall depth during interval i (mm) and E is the rainfall kinetic energy (MJ.ha⁻¹).

## SM2.6 Dataset-status coding and event inventory

Sediment-yield values were available for 46 of the 154 hydrological events in the laboratory-supplied dataset. Events without a reported usable sediment result were classified as “sediment yield unavailable” rather than as zero-export events. The available dataset did not allow the authors to distinguish between absent sampling, incomplete observations, laboratory rejection, or other possible causes of unavailability.

# Table SM3: Summary of rainfall-runoff events recorded between 2015 and 2023. For each event (ID), the table reports rainfall duration (h:min), rainfall depth observed (mm), runoff depth observed (mm), sediment yield (tonnes), rainfall kinetic energy E (MJ·ha⁻¹), maximum 30-min rainfall intensity I₃₀ (mm·h⁻¹).

| Event ID | Year | Month | Rainfall duration (h:min) | Rainfall depth observed (mm) | Runoff depth observed (mm) | Sediment Yield observed (tonnes) | Rainfall energy E (MJ.ha-1) | Max rainfall 30 min intensity I30 (mm.h-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 2015 | 1 | 8:00 | 18.00 | 1.19 |  | 2.36 | 5.20 |
| 1 | 2015 | 1 | 12:30 | 29.00 | 2.67 |  | 3.96 | 5.20 |
| 2 | 2015 | 1 | 4:40 | 6.20 | 0.16 | 0.42 | 0.93 | 4.40 |
| 3 | 2015 | 1 | 2:45 | 4.80 | 0.09 | 0.22 | 0.74 | 6.00 |
| 4 | 2015 | 1 | 3:05 | 4.40 | 0.16 | 0.15 | 0.53 | 2.80 |
| 5 | 2015 | 1 | 3:10 | 6.00 | 0.09 |  | 0.82 | 4.00 |
| 6 | 2015 | 6 | 1:35 | 18.60 | 0.76 |  | 4.77 | 21.60 |
| 7 | 2015 | 8 | 7:45 | 19.40 | 0.01 | 0.01 | 3.31 | 13.20 |
| 8 | 2016 | 1 | 11:50 | 17.00 | 0.98 |  | 2.16 | 4.40 |
| 9 | 2016 | 3 | 3:10 | 7.60 | 0.02 |  | 0.95 | 4.00 |
| 10 | 2016 | 4 | 3:30 | 5.60 | 0.00 |  | 0.78 | 3.60 |
| 11 | 2016 | 5 | 22:40 | 24.40 | 0.06 |  | 3.72 | 8.00 |
| 12 | 2016 | 6 | 1:20 | 5.80 | 0.03 |  | 1.09 | 9.20 |
| 13 | 2016 | 6 | 12:00 | 17.60 | 0.19 |  | 2.45 | 4.80 |
| 14 | 2016 | 6 | 0:25 | 4.80 | 0.01 |  | 0.85 | 7.20 |
| 15 | 2016 | 6 | 0:50 | 7.20 | 0.13 | 0.15 | 1.41 | 8.80 |
| 16 | 2016 | 6 | 2:00 | 8.40 | 0.06 |  | 1.64 | 10.00 |
| 17 | 2016 | 6 | 0:25 | 4.00 | 0.05 |  | 0.65 | 6.40 |
| 18 | 2016 | 6 | 5:30 | 6.20 | 0.02 |  | 1.11 | 6.00 |
| 19 | 2016 | 6 | 1:35 | 6.20 | 0.05 |  | 1.24 | 6.00 |
| 20 | 2016 | 6 | 4:20 | 6.80 | 0.12 |  | 0.81 | 2.40 |
| 21 | 2016 | 6 | 0:25 | 18.20 | 0.51 |  | 4.87 | 34.80 |
| 22 | 2016 | 6 | 0:05 | 2.60 | 0.02 |  | 0.55 | 5.20 |
| 23 | 2016 | 6 | 3:55 | 10.80 | 0.27 | 4.77 | 2.35 | 15.60 |
| 24 | 2016 | 11 | 7:45 | 10.00 | 0.01 |  | 1.30 | 4.40 |
| 25 | 2016 | 11 | 5:10 | 3.60 | 0.00 |  | 0.51 | 4.40 |
| 26 | 2016 | 11 | 2:45 | 5.00 | 0.01 |  | 0.66 | 3.20 |
| 27 | 2016 | 11 | 1:30 | 8.00 | 0.03 | 0.00 | 1.28 | 6.80 |
| 28 | 2017 | 1 | 4:20 | 5.60 | 0.08 |  | 0.69 | 2.80 |
| 29 | 2017 | 1 | 12:55 | 10.40 | 0.10 |  | 1.27 | 3.20 |
| 30 | 2017 | 1 | 5:55 | 9.00 | 0.36 |  | 1.10 | 2.40 |
| 31 | 2017 | 2 | 3:35 | 6.00 | 0.05 |  | 0.71 | 2.40 |
| 32 | 2017 | 2 | 1:25 | 6.20 | 0.01 |  | 1.11 | 8.40 |
| 33 | 2017 | 2 | 2:00 | 5.00 | 0.03 |  | 0.60 | 2.80 |
| 34 | 2017 | 3 | 0:05 | 1.80 | 0.02 |  | 0.23 | 2.40 |
| 35 | 2017 | 3 | 1:30 | 4.60 | 0.00 |  | 0.56 | 2.80 |
| 36 | 2017 | 3 | 14:20 | 10.00 | 0.01 |  | 1.19 | 2.40 |
| 37 | 2018 | 4 | 7:35 | 28.91 | 0.07 | 0.19 | 5.26 | 13.16 |
| 38 | 2018 | 6 | 5:10 | 6.23 | 0.15 |  | 0.83 | 2.18 |
| 39 | 2018 | 6 | 3:40 | 4.93 | 0.13 |  | 0.71 | 2.08 |
| 40 | 2018 | 8 | 17:45 | 26.82 | 0.08 | 1.08 | 6.01 | 19.16 |
| 41 | 2019 | 2 | 13:00 | 18.85 | 0.03 |  | 2.40 | 4.72 |
| 42 | 2019 | 3 | 4:40 | 8.75 | 0.03 |  | 0.96 | 2.46 |
| 43 | 2019 | 6 | 3:45 | 9.70 | 0.00 | 0.02 | 2.08 | 13.06 |
| 44 | 2019 | 6 | 0:15 | 12.91 | 0.14 | 1.80 | 3.48 | 24.74 |
| 45 | 2019 | 6 | 4:50 | 15.55 | 0.02 | 0.37 | 3.48 | 11.90 |
| 46 | 2019 | 8 | 14:20 | 34.88 | 0.31 | 0.48 | 6.58 | 18.26 |
| 47 | 2019 | 10 | 19:40 | 27.40 | 0.11 | 0.14 | 5.73 | 12.44 |
| 48 | 2019 | 10 | 2:55 | 4.86 | 0.01 | 0.00 | 0.84 | 7.28 |
| 49 | 2019 | 10 | 5:05 | 7.08 | 0.00 |  | 0.81 | 2.04 |
| 50 | 2019 | 11 | 5:05 | 3.27 | 0.03 | 0.04 | 0.35 | 1.94 |
| 51 | 2019 | 11 | 10:30 | 4.60 | 0.00 |  | 0.45 | 1.52 |
| 52 | 2019 | 12 | 3:50 | 4.81 | 0.02 | 0.06 | 0.63 | 3.54 |
| 53 | 2019 | 12 | 16:30 | 11.54 | 0.06 | 0.12 | 1.29 | 3.94 |
| 54 | 2019 | 12 | 17:40 | 13.80 | 0.01 |  | 1.61 | 3.76 |
| 55 | 2020 | 1 | 2:30 | 7.09 | 0.03 | 0.04 | 0.98 | 5.68 |
| 56 | 2020 | 2 | 5:00 | 18.79 | 0.01 |  | 2.61 | 5.16 |
| 57 | 2020 | 3 | 23:40 | 18.13 | 0.04 |  | 1.92 | 3.14 |
| 58 | 2020 | 3 | 4:40 | 4.62 | 0.00 |  | 0.50 | 1.66 |
| 59 | 2020 | 3 | 10:10 | 16.22 | 0.32 | 0.08 | 1.88 | 3.52 |
| 60 | 2020 | 3 | 5:20 | 5.85 | 0.28 | 0.11 | 0.62 | 1.80 |
| 61 | 2020 | 10 | 0:25 | 4.83 | 0.01 |  | 0.82 | 7.66 |
| 62 | 2020 | 10 | 5:00 | 4.49 | 0.01 |  | 0.49 | 2.14 |
| 63 | 2021 | 1 | 5:45 | 8.27 | 0.11 |  | 1.02 | 4.24 |
| 64 | 2021 | 1 | 0:15 | 3.97 | 0.07 |  | 0.80 | 6.50 |
| 65 | 2021 | 1 | 7:20 | 9.15 | 0.15 | 0.77 | 1.24 | 5.78 |
| 66 | 2021 | 1 | 5:25 | 10.77 | 0.12 |  | 1.66 | 4.88 |
| 67 | 2021 | 1 | 10:05 | 13.99 | 0.37 | 0.38 | 1.63 | 3.34 |
| 68 | 2021 | 1 | 11:05 | 7.80 | 0.38 |  | 0.92 | 3.32 |
| 69 | 2021 | 1 | 9:30 | 7.04 | 0.38 | 0.07 | 0.71 | 1.40 |
| 70 | 2021 | 2 | 4:20 | 12.99 | 1.07 | 5.31 | 1.99 | 9.32 |
| 71 | 2021 | 3 | 10:05 | 12.48 | 0.22 |  | 1.62 | 4.48 |
| 72 | 2021 | 3 | 8:05 | 7.25 | 0.20 |  | 0.95 | 3.54 |
| 73 | 2021 | 3 | 13:15 | 6.12 | 0.16 |  | 0.59 | 1.22 |
| 74 | 2021 | 4 | 18:05 | 22.59 | 0.29 |  | 2.67 | 5.48 |
| 75 | 2021 | 5 | 1:00 | 2.25 | 0.01 |  | 0.25 | 1.12 |
| 76 | 2021 | 5 | 0:15 | 6.63 | 0.00 |  | 1.58 | 13.12 |
| 77 | 2021 | 5 | 3:55 | 5.52 | 0.02 |  | 0.65 | 2.68 |
| 78 | 2021 | 5 | 3:05 | 9.21 | 0.14 |  | 1.65 | 7.08 |
| 79 | 2021 | 5 | 4:05 | 7.82 | 0.16 | 0.10 | 1.39 | 9.92 |
| 80 | 2021 | 6 | 1:35 | 7.55 | 0.28 | 5.45 | 1.41 | 9.88 |
| 81 | 2021 | 6 | 2:20 | 14.37 | 0.49 |  | 3.64 | 17.02 |
| 82 | 2021 | 6 | 1:35 | 3.35 | 0.03 |  | 0.48 | 4.74 |
| 83 | 2021 | 6 | 9:30 | 35.31 | 3.33 |  | 5.51 | 11.58 |
| 84 | 2021 | 6 | 13:30 | 37.01 | 2.72 |  | 5.53 | 8.32 |
| 85 | 2021 | 7 | 2:00 | 5.95 | 0.20 |  | 0.98 | 5.18 |
| 86 | 2021 | 7 | 6:25 | 11.39 | 0.52 |  | 2.28 | 13.42 |
| 87 | 2021 | 7 | 1:10 | 19.24 | 1.48 |  | 4.96 | 30.32 |
| 88 | 2021 | 7 | 21:05 | 45.51 | 2.97 |  | 9.27 | 25.20 |
| 89 | 2021 | 8 | 2:10 | 10.85 | 0.36 |  | 1.65 | 6.72 |
| 90 | 2021 | 8 | 8:20 | 11.78 | 0.31 |  | 2.42 | 13.48 |
| 91 | 2021 | 8 | 3:00 | 10.88 | 0.44 |  | 2.69 | 11.82 |
| 92 | 2021 | 8 | 6:50 | 4.76 | 0.12 |  | 1.02 | 4.20 |
| 93 | 2021 | 8 | 0:10 | 7.57 | 0.47 |  | 2.03 | 14.86 |
| 94 | 2021 | 8 | 0:30 | 3.08 | 0.07 |  | 0.41 | 3.86 |
| 95 | 2021 | 8 | 2:00 | 5.26 | 0.07 |  | 0.85 | 4.74 |
| 96 | 2021 | 8 | 3:45 | 4.41 | 0.08 |  | 0.52 | 2.52 |
| 97 | 2021 | 9 | 2:00 | 8.40 | 0.08 |  | 1.32 | 6.40 |
| 98 | 2021 | 10 | 2:45 | 3.40 | 0.02 |  | 0.42 | 2.80 |
| 99 | 2021 | 10 | 20:05 | 32.20 | 0.25 |  | 4.75 | 7.60 |
| 100 | 2021 | 10 | 2:20 | 4.60 | 0.05 |  | 0.62 | 2.80 |
| 101 | 2021 | 10 | 6:30 | 3.40 | 0.03 |  | 0.40 | 0.80 |
| 102 | 2021 | 10 | 3:15 | 2.80 | 0.03 |  | 0.33 | 1.60 |
| 103 | 2021 | 10 | 10:25 | 4.47 | 0.03 |  | 0.57 | 2.98 |
| 104 | 2021 | 10 | 1:55 | 3.79 | 0.04 |  | 0.52 | 4.34 |
| 105 | 2021 | 10 | 3:10 | 5.51 | 0.02 |  | 0.78 | 3.74 |
| 106 | 2021 | 11 | 6:35 | 3.30 | 0.06 |  | 0.32 | 1.20 |
| 107 | 2021 | 12 | 3:50 | 3.36 | 0.03 |  | 0.34 | 1.76 |
| 108 | 2021 | 12 | 5:10 | 3.76 | 0.02 |  | 0.49 | 3.02 |
| 109 | 2021 | 12 | 8:05 | 7.90 | 0.10 |  | 0.99 | 3.22 |
| 110 | 2021 | 12 | 3:10 | 7.75 | 0.08 |  | 0.94 | 3.34 |
| 111 | 2021 | 12 | 23:35 | 15.89 | 0.23 |  | 1.83 | 3.58 |
| 112 | 2021 | 12 | 6:40 | 18.69 | 0.22 | 0.30 | 2.72 | 8.36 |
| 113 | 2021 | 12 | 9:10 | 8.06 | 0.11 |  | 0.94 | 3.30 |
| 114 | 2022 | 1 | 0:50 | 10.66 | 0.11 |  | 2.14 | 11.18 |
| 115 | 2022 | 1 | 17:35 | 14.01 | 0.57 |  | 1.71 | 4.44 |
| 116 | 2022 | 1 | 12:25 | 16.02 | 1.16 |  | 1.82 | 3.18 |
| 117 | 2022 | 2 | 11:40 | 13.54 | 0.09 | 0.03 | 1.61 | 3.70 |
| 118 | 2022 | 2 | 10:00 | 4.24 | 0.00 |  | 0.43 | 1.68 |
| 119 | 2022 | 2 | 1:10 | 7.00 | 0.19 | 0.84 | 1.36 | 9.60 |
| 120 | 2022 | 2 | 4:05 | 3.80 | 0.05 |  | 0.45 | 1.20 |
| 121 | 2022 | 5 | 3:40 | 18.64 | 1.04 | 18.15 | 4.46 | 26.68 |
| 122 | 2022 | 6 | 3:20 | 15.18 | 0.07 | 0.02 | 2.54 | 10.82 |
| 123 | 2022 | 6 | 5:15 | 19.50 | 1.51 | 15.84 | 4.97 | 30.10 |
| 124 | 2022 | 6 | 4:40 | 14.70 | 1.13 | 0.91 | 1.91 | 4.76 |
| 125 | 2022 | 6 | 2:00 | 13.75 | 0.02 |  | 2.30 | 7.96 |
| 126 | 2022 | 6 | 8:45 | 7.70 | 0.02 | 0.49 | 1.43 | 8.56 |
| 127 | 2022 | 6 | 1:30 | 12.05 | 0.05 | 0.11 | 2.33 | 11.98 |
| 128 | 2022 | 9 | 4:45 | 13.36 | 0.08 |  | 1.81 | 5.24 |
| 129 | 2022 | 12 | 5:35 | 8.83 | 0.02 |  | 1.07 | 4.58 |
| 130 | 2022 | 12 | 4:40 | 11.32 | 0.05 |  | 1.59 | 7.14 |
| 131 | 2022 | 12 | 12:30 | 7.50 | 0.07 |  | 0.85 | 3.08 |
| 132 | 2023 | 1 | 13:15 | 12.72 | 0.01 |  | 1.53 | 5.26 |
| 133 | 2023 | 3 | 19:25 | 14.04 | 0.09 |  | 1.62 | 4.96 |
| 134 | 2023 | 5 | 5:10 | 7.57 | 0.18 | 0.07 | 0.85 | 2.54 |
| 135 | 2023 | 6 | 1:40 | 20.56 | 0.12 | 0.09 | 5.06 | 19.24 |
| 136 | 2023 | 7 | 1:05 | 8.16 | 0.10 |  | 1.87 | 12.96 |
| 137 | 2023 | 7 | 0:15 | 8.71 | 0.37 |  | 2.19 | 16.82 |
| 138 | 2023 | 8 | 10:30 | 7.37 | 0.00 |  | 1.18 | 7.80 |
| 139 | 2023 | 8 | 5:05 | 11.32 | 0.01 |  | 1.91 | 10.04 |
| 140 | 2023 | 9 | 7:45 | 11.35 | 0.01 |  | 1.71 | 6.52 |
| 141 | 2023 | 10 | 5:00 | 11.55 | 0.01 |  | 1.49 | 5.24 |
| 142 | 2023 | 10 | 2:40 | 5.57 | 0.02 |  | 1.20 | 9.46 |
| 143 | 2023 | 11 | 10:50 | 13.42 | 0.09 | 0.13 | 2.69 | 9.94 |
| 144 | 2023 | 11 | 15:25 | 12.93 | 0.04 |  | 1.56 | 3.84 |
| 145 | 2023 | 11 | 6:25 | 7.83 | 0.06 | 0.05 | 1.07 | 4.60 |
| 146 | 2023 | 11 | 9:55 | 11.79 | 0.08 | 0.15 | 1.80 | 7.98 |
| 147 | 2023 | 11 | 7:30 | 6.08 | 0.03 |  | 0.66 | 2.76 |
| 148 | 2023 | 11 | 8:15 | 14.21 | 0.22 | 0.12 | 1.98 | 6.48 |
| 149 | 2023 | 11 | 16:30 | 13.01 | 0.06 |  | 1.30 | 1.70 |
| 150 | 2023 | 12 | 23:20 | 18.04 | 0.92 | 0.31 | 2.10 | 5.68 |
| 151 | 2023 | 12 | 6:55 | 7.50 | 0.44 | 2.68 | 0.83 | 3.12 |
| 152 | 2023 | 12 | 7:40 | 10.31 | 0.90 | 0.19 | 1.08 | 2.28 |
| 153 | 2023 | 12 | 8:00 | 5.79 | 0.56 | 0.58 | 0.75 | 3.72 |

Table SM3b: Rainfall events excluded from the final dataset, including the month, rain duration, and observed rainfall and runoff depths.

|  | Year | Month | Rain duration (h:min) | Rainfall depth observed (mm) | Runoff depth observed (mm) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2021 | 2 | 2:05 | 3.84 | 0.34 |  |
|  | 2021 | 6 | 1:30 | 31.16 | 2.20 |  |
|  | 2021 | 6 | 1:20 | 3.48 | 0.16 |  |
|  | 2021 | 6 | 0:15 | 3.97 | 1.02 |  |
|  | 2021 | 7 | 6:05 | 4.02 | 0.20 |  |
|  | 2021 | 7 | 24:10 | 74.84 | 8.50 |  |
|  | 2021 | 8 | 0:20 | 3.58 | 0.22 |  |
|  | 2022 | 1 | 1:55 | 5.91 | 0.41 |  |
|  | 2023 | 12 | 0:30 | 4.40 | 0.22 |  |
|  | 2023 | 12 | 0:25 | 3.72 | 0.14 |  |
|  | 2023 | 12 | 4:20 | 2.58 | 0.14 |  |
|  | 2023 | 12 | 1:20 | 2.63 | 0.19 |  |
|  | 2023 | 12 | 8:45 | 8.13 | 0.82 |  |
|  | 2023 | 12 | 3:10 | 3.34 | 0.53 |  |
|  | 2023 | 12 | 3:20 | 4.90 | 0.33 |  |

# SM3.	Benchmark fixed- and literature-based λ formulations

Several fixed- and literature-based λ formulations were tested as benchmarks within the SCS-CN framework. These included the traditional value λ=0.20, corresponding to the original SCS assumption (USDA-NRCS, 2021); the revised value λ=0.05, recommended following the reanalysis of event-based rainfall-runoff datasets (Woodward et al., 2003); and a lower benchmark value of λ=0.01, motivated by studies reporting maximum model efficiency or observed initial abstraction ratios of approximately this order of magnitude (Mishra and Singh, 2004; Baltas et al., 2007).

For the fixed benchmark simulations, λ=0.05 and λ=0.01 were applied directly using the original Curve Number values derived under the conventional λ=0.20 assumption. The baseline CN values were therefore neither adjusted nor recalibrated when changing the initial abstraction ratio. These simulations were intended to isolate the effect of modifying λ, rather than to represent fully recalibrated SCS-CN parameter sets.

Published alternative parameterisations were also evaluated:

- Woodward et al. (2003):	S0.05=0.8187 S0.21.15		with λ=0.05

- Hawkins et al. (2008):		S0.05=1.42 S0.2		with λ=0.05

- Jain et al. (2006):		Ia= λ S0.2 PP+S0.2α 		with λ=0.3 and α=1.5

where P is the rainfall depth (mm).

Table SM4: Observed rainfall, observed runoff, annual mean Curve Number, and runoff depths predicted using fixed and literature-based SCS–CN initial abstraction formulations. Predictions are reported for fixed λ values of 0.20, 0.05, and 0.01, and for the Woodward, Hawkins, and Jain formulations. Zero values indicate events for which the formulation did not generate runoff.

| Event ID |  | Observed |  |  |  | Runoff Predicted (mm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rainfall (mm) | Runoff (mm) | CN mean |  | λ = 0.2 | λ = 0.05 | λ = 0.01 | Woodward | Hawkins | Jain |
| 0 |  | 18 | 1.19 | 72.71 |  | 0 | 1.61 | 2.59 | 1.00 | 1.18 | 2.35 |
| 1 |  | 29 | 2.67 | 72.71 |  | 0.94 | 4.91 | 6.38 | 3.88 | 4.20 | 5.49 |
| 2 |  | 6.2 | 0.16 | 72.71 |  | 0 | 0.02 | 0.27 | 0 | 0 | 0.33 |
| 3 |  | 4.8 | 0.09 | 72.71 |  | 0 | 0.00 | 0.15 | 0 | 0 | 0.20 |
| 4 |  | 4.4 | 0.16 | 72.71 |  | 0 | 0 | 0.12 | 0 | 0 | 0.17 |
| 5 |  | 6 | 0.09 | 72.71 |  | 0 | 0.02 | 0.25 | 0 | 0 | 0.31 |
| 6 |  | 18.6 | 0.76 | 72.71 |  | 0 | 1.75 | 2.76 | 1.11 | 1.31 | 2.49 |
| 7 |  | 19.4 | 0.01 | 72.71 |  | 0.00 | 1.95 | 2.99 | 1.27 | 1.48 | 2.69 |
| 8 |  | 17 | 0.98 | 73.87 |  | 0 | 1.53 | 2.45 | 0.96 | 1.12 | 2.22 |
| 9 |  | 7.6 | 0.02 | 73.87 |  | 0 | 0.10 | 0.47 | 0.00 | 0.02 | 0.51 |
| 10 |  | 5.6 | 0.00 | 73.87 |  | 0 | 0.01 | 0.23 | 0.00 | 0.00 | 0.29 |
| 11 |  | 24.4 | 0.06 | 73.87 |  | 0.43 | 3.61 | 4.87 | 2.76 | 3.01 | 4.24 |
| 12 |  | 5.8 | 0.03 | 73.87 |  | 0 | 0.02 | 0.25 | 0.00 | 0.00 | 0.31 |
| 13 |  | 17.6 | 0.19 | 73.87 |  | 0 | 1.67 | 2.62 | 1.07 | 1.25 | 2.37 |
| 14 |  | 4.8 | 0.01 | 73.87 |  | 0 | 0.00 | 0.16 | 0 | 0 | 0.21 |
| 15 |  | 7.2 | 0.13 | 73.87 |  | 0 | 0.08 | 0.41 | 0 | 0.01 | 0.46 |
| 16 |  | 8.4 | 0.06 | 73.87 |  | 0 | 0.16 | 0.58 | 0.02 | 0.04 | 0.61 |
| 17 |  | 4 | 0.05 | 73.87 |  | 0 | 0 | 0.10 | 0 | 0 | 0.15 |
| 18 |  | 6.2 | 0.02 | 73.87 |  | 0 | 0.03 | 0.30 | 0 | 0 | 0.35 |
| 19 |  | 6.2 | 0.05 | 73.87 |  | 0 | 0.03 | 0.30 | 0 | 0 | 0.35 |
| 20 |  | 6.8 | 0.12 | 73.87 |  | 0 | 0.06 | 0.36 | 0 | 0.00 | 0.41 |
| 21 |  | 18.2 | 0.51 | 73.87 |  | 0.00 | 1.81 | 2.79 | 1.20 | 1.37 | 2.51 |
| 22 |  | 2.6 | 0.02 | 73.87 |  | 0 | 0 | 0.03 | 0.00 | 0.00 | 0.07 |
| 23 |  | 10.8 | 0.27 | 73.87 |  | 0 | 0.41 | 0.98 | 0.14 | 0.21 | 0.97 |
| 24 |  | 10 | 0.01 | 73.87 |  | 0 | 0.32 | 0.84 | 0.08 | 0.14 | 0.84 |
| 25 |  | 3.6 | 0.00 | 73.87 |  | 0 | 0 | 0.08 | 0 | 0 | 0.12 |
| 26 |  | 5 | 0.01 | 73.87 |  | 0 | 0.00 | 0.18 | 0 | 0 | 0.23 |
| 27 |  | 8 | 0.03 | 73.87 |  | 0 | 0.13 | 0.52 | 0.01 | 0.03 | 0.56 |
| 28 |  | 5.6 | 0.08 | 72.87 |  | 0 | 0.01 | 0.22 | 0 | 0 | 0.27 |
| 29 |  | 10.4 | 0.10 | 72.87 |  | 0 | 0.32 | 0.86 | 0.08 | 0.14 | 0.87 |
| 30 |  | 9 | 0.36 | 72.87 |  | 0 | 0.18 | 0.63 | 0.02 | 0.05 | 0.67 |
| 31 |  | 6 | 0.05 | 72.87 |  | 0 | 0.02 | 0.26 | 0 | 0 | 0.31 |
| 32 |  | 6.2 | 0.01 | 72.87 |  | 0 | 0.02 | 0.28 | 0 | 0 | 0.33 |
| 33 |  | 5 | 0.03 | 72.87 |  | 0 | 0.00 | 0.17 | 0 | 0 | 0.22 |
| 34 |  | 1.8 | 0.02 | 72.87 |  | 0 | 0 | 0.01 | 0 | 0 | 0.03 |
| 35 |  | 4.6 | 0.00 | 72.87 |  | 0 | 0 | 0.14 | 0 | 0 | 0.19 |
| 36 |  | 10 | 0.01 | 72.87 |  | 0 | 0.28 | 0.79 | 0.06 | 0.11 | 0.81 |
| 37 |  | 28.91 | 0.07 | 74.56 |  | 1.36 | 5.43 | 6.86 | 4.45 | 4.73 | 5.86 |
| 38 |  | 6.23 | 0.15 | 74.56 |  | 0 | 0.04 | 0.31 | 0 | 0.00 | 0.36 |
| 39 |  | 4.93 | 0.13 | 74.56 |  | 0 | 0.00 | 0.18 | 0 | 0 | 0.23 |
| 40 |  | 26.82 | 0.08 | 74.56 |  | 0.94 | 4.63 | 5.98 | 3.71 | 3.98 | 5.14 |
| 41 |  | 18.85 | 0.03 | 73.34 |  | 0.00 | 1.90 | 2.91 | 1.25 | 1.44 | 2.62 |
| 42 |  | 8.75 | 0.03 | 73.34 |  | 0 | 0.18 | 0.61 | 0.02 | 0.05 | 0.64 |
| 43 |  | 9.7 | 0.00 | 73.34 |  | 0 | 0.27 | 0.76 | 0.05 | 0.10 | 0.78 |
| 44 |  | 12.91 | 0.14 | 73.34 |  | 0 | 0.68 | 1.38 | 0.30 | 0.41 | 1.32 |
| 45 |  | 15.55 | 0.02 | 73.34 |  | 0 | 1.16 | 2.00 | 0.65 | 0.80 | 1.85 |
| 46 |  | 34.88 | 0.31 | 73.34 |  | 2.48 | 7.47 | 9.13 | 6.28 | 6.65 | 7.75 |
| 47 |  | 27.40 | 0.11 | 73.34 |  | 0.79 | 4.51 | 5.90 | 3.54 | 3.84 | 5.09 |
| 48 |  | 4.86 | 0.01 | 73.34 |  | 0 | 0.00 | 0.16 | 0 | 0 | 0.21 |
| 49 |  | 7.08 | 0.00 | 73.34 |  | 0 | 0.06 | 0.38 | 0 | 0.00 | 0.43 |
| 50 |  | 3.27 | 0.03 | 73.34 |  | 0 | 0 | 0.06 | 0 | 0 | 0.10 |
| 51 |  | 4.60 | 0.00 | 73.34 |  | 0 | 0 | 0.14 | 0 | 0 | 0.19 |
| 52 |  | 4.81 | 0.02 | 73.34 |  | 0 | 0.00 | 0.16 | 0 | 0 | 0.21 |
| 53 |  | 11.54 | 0.06 | 73.34 |  | 0 | 0.48 | 1.09 | 0.17 | 0.26 | 1.07 |
| 54 |  | 13.80 | 0.01 | 73.34 |  | 0 | 0.83 | 1.58 | 0.41 | 0.53 | 1.49 |
| 55 |  | 7.09 | 0.03 | 74.21 |  | 0 | 0.08 | 0.41 | 0.00 | 0.01 | 0.45 |
| 56 |  | 18.79 | 0.01 | 74.21 |  | 0.01 | 2.01 | 3.02 | 1.37 | 1.56 | 2.70 |
| 57 |  | 18.13 | 0.04 | 74.21 |  | 0.00 | 1.84 | 2.82 | 1.23 | 1.41 | 2.53 |
| 58 |  | 4.62 | 0.00 | 74.21 |  | 0 | 0.00 | 0.15 | 0 | 0 | 0.20 |
| 59 |  | 16.22 | 0.32 | 74.21 |  | 0 | 1.39 | 2.27 | 0.86 | 1.01 | 2.07 |
| 60 |  | 5.85 | 0.28 | 74.21 |  | 0 | 0.02 | 0.26 | 0 | 0 | 0.32 |
| 61 |  | 4.83 | 0.01 | 74.21 |  | 0 | 0.00 | 0.17 | 0 | 0 | 0.22 |
| 62 |  | 4.49 | 0.01 | 74.21 |  | 0 | 0.00 | 0.14 | 0 | 0 | 0.19 |
| 63 |  | 8.27 | 0.11 | 72.85 |  | 0 | 0.13 | 0.53 | 0.00 | 0.02 | 0.57 |
| 64 |  | 3.97 | 0.07 | 72.85 |  | 0 | 0 | 0.09 | 0 | 0 | 0.14 |
| 65 |  | 9.15 | 0.15 | 72.85 |  | 0 | 0.20 | 0.65 | 0.02 | 0.06 | 0.69 |
| 66 |  | 10.77 | 0.12 | 72.85 |  | 0 | 0.36 | 0.92 | 0.10 | 0.17 | 0.93 |
| 67 |  | 13.99 | 0.37 | 72.85 |  | 0 | 0.82 | 1.58 | 0.40 | 0.52 | 1.50 |
| 68 |  | 7.80 | 0.38 | 72.85 |  | 0 | 0.10 | 0.46 | 0.00 | 0.01 | 0.51 |
| 69 |  | 7.04 | 0.38 | 72.85 |  | 0 | 0.05 | 0.37 | 0 | 0.00 | 0.42 |
| 70 |  | 12.99 | 1.07 | 72.85 |  | 0 | 0.66 | 1.36 | 0.28 | 0.39 | 1.31 |
| 71 |  | 12.48 | 0.22 | 72.85 |  | 0 | 0.59 | 1.25 | 0.23 | 0.33 | 1.22 |
| 72 |  | 7.25 | 0.20 | 72.85 |  | 0 | 0.07 | 0.39 | 0 | 0.00 | 0.44 |
| 73 |  | 6.12 | 0.16 | 72.85 |  | 0 | 0.02 | 0.27 | 0 | 0 | 0.32 |
| 74 |  | 22.59 | 0.29 | 72.85 |  | 0.14 | 2.83 | 4.03 | 2.03 | 2.28 | 3.55 |
| 75 |  | 2.25 | 0.01 | 72.85 |  | 0 | 0 | 0.02 | 0 | 0 | 0.05 |
| 76 |  | 6.63 | 0.00 | 72.85 |  | 0 | 0.04 | 0.32 | 0 | 0 | 0.38 |
| 77 |  | 5.52 | 0.02 | 72.85 |  | 0 | 0.01 | 0.21 | 0 | 0 | 0.27 |
| 78 |  | 9.21 | 0.14 | 72.85 |  | 0 | 0.20 | 0.66 | 0.02 | 0.06 | 0.69 |
| 79 |  | 7.82 | 0.16 | 72.85 |  | 0 | 0.10 | 0.47 | 0.00 | 0.01 | 0.51 |
| 80 |  | 7.55 | 0.28 | 72.85 |  | 0 | 0.08 | 0.43 | 0 | 0.01 | 0.48 |
| 81 |  | 14.37 | 0.49 | 72.85 |  | 0 | 0.89 | 1.67 | 0.44 | 0.57 | 1.57 |
| 82 |  | 3.35 | 0.03 | 72.85 |  | 0 | 0 | 0.06 | 0 | 0 | 0.10 |
| 83 |  | 35.31 | 3.33 | 72.85 |  | 2.42 | 7.46 | 9.15 | 6.25 | 6.63 | 7.77 |
| 84 |  | 37.01 | 2.72 | 72.85 |  | 2.90 | 8.21 | 9.95 | 6.94 | 7.34 | 8.43 |
| 85 |  | 5.95 | 0.20 | 72.85 |  | 0 | 0.02 | 0.25 | 0 | 0 | 0.31 |
| 86 |  | 11.39 | 0.52 | 72.85 |  | 0 | 0.44 | 1.04 | 0.14 | 0.22 | 1.03 |
| 87 |  | 19.24 | 1.48 | 72.85 |  | 0.00 | 1.93 | 2.96 | 1.26 | 1.46 | 2.66 |
| 88 |  | 45.51 | 2.97 | 72.85 |  | 5.83 | 12.28 | 14.26 | 10.81 | 11.27 | 12.01 |
| 89 |  | 10.85 | 0.36 | 72.85 |  | 0 | 0.37 | 0.94 | 0.10 | 0.17 | 0.94 |
| 90 |  | 11.78 | 0.31 | 72.85 |  | 0 | 0.49 | 1.11 | 0.17 | 0.26 | 1.09 |
| 91 |  | 10.88 | 0.44 | 72.85 |  | 0 | 0.37 | 0.94 | 0.11 | 0.17 | 0.94 |
| 92 |  | 4.76 | 0.12 | 72.85 |  | 0 | 0.00 | 0.15 | 0 | 0 | 0.20 |
| 93 |  | 7.57 | 0.47 | 72.85 |  | 0 | 0.08 | 0.43 | 0 | 0.01 | 0.48 |
| 94 |  | 3.08 | 0.07 | 72.85 |  | 0 | 0 | 0.05 | 0 | 0 | 0.09 |
| 95 |  | 5.26 | 0.07 | 72.85 |  | 0 | 0.00 | 0.19 | 0 | 0 | 0.24 |
| 96 |  | 4.41 | 0.08 | 72.85 |  | 0 | 0 | 0.12 | 0 | 0 | 0.17 |
| 97 |  | 8.40 | 0.08 | 72.85 |  | 0 | 0.14 | 0.54 | 0.01 | 0.03 | 0.58 |
| 98 |  | 3.40 | 0.02 | 72.85 |  | 0 | 0 | 0.06 | 0 | 0 | 0.11 |
| 99 |  | 32.20 | 0.25 | 72.85 |  | 1.63 | 6.18 | 7.76 | 5.05 | 5.40 | 6.62 |
| 100 |  | 4.60 | 0.05 | 72.85 |  | 0 | 0 | 0.14 | 0 | 0 | 0.19 |
| 101 |  | 3.40 | 0.03 | 72.85 |  | 0 | 0 | 0.06 | 0 | 0 | 0.11 |
| 102 |  | 2.80 | 0.03 | 72.85 |  | 0 | 0 | 0.04 | 0 | 0 | 0.07 |
| 103 |  | 4.47 | 0.03 | 72.85 |  | 0 | 0 | 0.13 | 0 | 0 | 0.18 |
| 104 |  | 3.79 | 0.04 | 72.85 |  | 0 | 0 | 0.08 | 0 | 0 | 0.13 |
| 105 |  | 5.51 | 0.02 | 72.85 |  | 0 | 0.01 | 0.21 | 0 | 0 | 0.27 |
| 106 |  | 3.30 | 0.06 | 72.85 |  | 0 | 0 | 0.06 | 0 | 0 | 0.10 |
| 107 |  | 3.36 | 0.03 | 72.85 |  | 0 | 0 | 0.06 | 0 | 0 | 0.10 |
| 108 |  | 3.76 | 0.02 | 72.85 |  | 0 | 0 | 0.08 | 0 | 0 | 0.13 |
| 109 |  | 7.90 | 0.10 | 72.85 |  | 0 | 0.10 | 0.48 | 0.00 | 0.01 | 0.52 |
| 110 |  | 7.75 | 0.08 | 72.85 |  | 0 | 0.09 | 0.46 | 0.00 | 0.01 | 0.50 |
| 111 |  | 15.89 | 0.23 | 72.85 |  | 0 | 1.18 | 2.04 | 0.66 | 0.81 | 1.89 |
| 112 |  | 18.69 | 0.22 | 72.85 |  | 0 | 1.79 | 2.80 | 1.15 | 1.34 | 2.53 |
| 113 |  | 8.06 | 0.11 | 72.85 |  | 0 | 0.11 | 0.50 | 0.00 | 0.02 | 0.54 |
| 114 |  | 10.66 | 0.11 | 73.75 |  | 0 | 0.39 | 0.95 | 0.12 | 0.19 | 0.95 |
| 115 |  | 14.01 | 0.57 | 73.75 |  | 0 | 0.90 | 1.66 | 0.47 | 0.59 | 1.56 |
| 116 |  | 16.02 | 1.16 | 73.75 |  | 0 | 1.30 | 2.17 | 0.77 | 0.92 | 1.99 |
| 117 |  | 13.54 | 0.09 | 73.75 |  | 0 | 0.82 | 1.55 | 0.41 | 0.52 | 1.47 |
| 118 |  | 4.24 | 0.00 | 73.75 |  | 0 | 0 | 0.12 | 0 | 0 | 0.17 |
| 119 |  | 7.00 | 0.19 | 73.75 |  | 0 | 0.07 | 0.39 | 0 | 0.00 | 0.43 |
| 120 |  | 3.80 | 0.05 | 73.75 |  | 0 | 0 | 0.09 | 0 | 0 | 0.14 |
| 121 |  | 18.64 | 1.04 | 73.75 |  | 0.00 | 1.91 | 2.91 | 1.27 | 1.46 | 2.61 |
| 122 |  | 15.18 | 0.07 | 73.75 |  | 0 | 1.12 | 1.95 | 0.64 | 0.77 | 1.80 |
| 123 |  | 19.50 | 1.51 | 73.75 |  | 0.02 | 2.13 | 3.17 | 1.46 | 1.65 | 2.83 |
| 124 |  | 14.70 | 1.13 | 73.75 |  | 0 | 1.03 | 1.83 | 0.56 | 0.69 | 1.70 |
| 125 |  | 13.75 | 0.02 | 73.75 |  | 0 | 0.85 | 1.60 | 0.43 | 0.55 | 1.51 |
| 126 |  | 7.70 | 0.02 | 73.75 |  | 0 | 0.11 | 0.48 | 0.00 | 0.02 | 0.52 |
| 127 |  | 12.05 | 0.05 | 73.75 |  | 0 | 0.58 | 1.22 | 0.24 | 0.33 | 1.18 |
| 128 |  | 13.36 | 0.08 | 73.75 |  | 0 | 0.79 | 1.51 | 0.38 | 0.49 | 1.43 |
| 129 |  | 8.83 | 0.02 | 73.75 |  | 0 | 0.20 | 0.64 | 0.03 | 0.06 | 0.67 |
| 130 |  | 11.32 | 0.05 | 73.75 |  | 0 | 0.48 | 1.08 | 0.17 | 0.25 | 1.06 |
| 131 |  | 7.50 | 0.07 | 73.75 |  | 0 | 0.10 | 0.45 | 0.00 | 0.01 | 0.49 |
| 132 |  | 12.72 | 0.01 | 72.89 |  | 0 | 0.62 | 1.30 | 0.26 | 0.36 | 1.26 |
| 133 |  | 14.04 | 0.09 | 72.89 |  | 0 | 0.84 | 1.59 | 0.40 | 0.53 | 1.51 |
| 134 |  | 7.57 | 0.18 | 72.89 |  | 0 | 0.08 | 0.43 | 0 | 0.01 | 0.48 |
| 135 |  | 20.56 | 0.12 | 72.89 |  | 0.03 | 2.27 | 3.37 | 1.55 | 1.77 | 3.00 |
| 136 |  | 8.16 | 0.10 | 72.89 |  | 0 | 0.12 | 0.51 | 0.00 | 0.02 | 0.56 |
| 137 |  | 8.71 | 0.37 | 72.89 |  | 0 | 0.16 | 0.59 | 0.01 | 0.04 | 0.63 |
| 138 |  | 7.37 | 0.00 | 72.89 |  | 0 | 0.07 | 0.41 | 0 | 0.00 | 0.46 |
| 139 |  | 11.32 | 0.01 | 72.89 |  | 0 | 0.43 | 1.03 | 0.14 | 0.21 | 1.02 |
| 140 |  | 11.35 | 0.01 | 72.89 |  | 0 | 0.43 | 1.03 | 0.14 | 0.22 | 1.02 |
| 141 |  | 11.55 | 0.01 | 72.89 |  | 0 | 0.46 | 1.07 | 0.15 | 0.24 | 1.06 |
| 142 |  | 5.57 | 0.02 | 72.89 |  | 0 | 0.01 | 0.22 | 0 | 0 | 0.27 |
| 143 |  | 13.42 | 0.09 | 72.89 |  | 0 | 0.73 | 1.46 | 0.33 | 0.45 | 1.39 |
| 144 |  | 12.93 | 0.04 | 72.89 |  | 0 | 0.66 | 1.35 | 0.28 | 0.38 | 1.30 |
| 145 |  | 7.83 | 0.06 | 72.89 |  | 0 | 0.10 | 0.47 | 0.00 | 0.01 | 0.51 |
| 146 |  | 11.79 | 0.08 | 72.89 |  | 0 | 0.49 | 1.12 | 0.17 | 0.26 | 1.10 |
| 147 |  | 6.08 | 0.03 | 72.89 |  | 0 | 0.02 | 0.26 | 0 | 0 | 0.32 |
| 148 |  | 14.21 | 0.22 | 72.89 |  | 0 | 0.87 | 1.63 | 0.43 | 0.55 | 1.54 |
| 149 |  | 13.01 | 0.06 | 72.89 |  | 0 | 0.67 | 1.37 | 0.29 | 0.39 | 1.31 |
| 150 |  | 18.04 | 0.92 | 72.89 |  | 0 | 1.65 | 2.62 | 1.03 | 1.21 | 2.38 |
| 151 |  | 7.50 | 0.44 | 72.89 |  | 0 | 0.08 | 0.43 | 0 | 0.01 | 0.47 |
| 152 |  | 10.31 | 0.90 | 72.89 |  | 0 | 0.31 | 0.84 | 0.07 | 0.13 | 0.86 |
| 153 |  | 5.79 | 0.56 | 72.89 |  | 0 | 0.01 | 0.24 | 0 | 0 | 0.39 |

SM4.	Detailed formulation of erosion factors and parameterisation

This section provides the detailed parameterisation of the erosion and hydrological factors used in the MUSLE calculations. It describes the calculation of the soil-erodibility factor, the assumptions adopted for soil structure and permeability, the derivation of the cover-management and support-practice factors, and the grid-based calculation of the topographic factor. It also discusses the influence of DEM resolution on the LS factor and reports the annual Curve Number and cover-management values. Tables SM5, SM6, and SM6b summarise the fixed assumptions, catchment-scale MUSLE inputs, and annual hydrological and land-cover parameters used in the analysis.

## SM4.1 	Soil erodibility factor (K)

SM4.1.1	 Nomograph-based formulation

The soil erodibility factor (K in t·ha·h·ha⁻¹·MJ⁻¹·mm⁻¹) was computed using the empirical formulation derived from the original nomograph developed by Wischmeier and Smith (1971) and later formalised for computational use by Foster et al. (1981). The following equation was applied:

K=2.7668×10-7 M1.14(12-a)+4.2819×10-3(b-2)+3.2938×10-3(c-3)		
where

a is soil organic matter content (%),

b is the soil structure class (dimensionless),

c is the soil permeability class (dimensionless),

M is a combined textural index.

SM4.1.2	 Textural index M

The textural index M accounts for the susceptibility of fine particles to detachment and transport and was computed following the piecewise formulation reported in the original nomograph-based approach and subsequent refinements:

M=t'(t'+d'),for t'≤67%67(67+d')+(t'-67)[-2.416d'-17.03t'+0.02491d'2+0.1103t'2+0.03238t'd'+730.8],for t'>67% and M≤8000
where

t' is the percentage of silt plus very fine sand (0.002–0.1 mm),

d' is the percentage of non-very-fine sand (> 0.1 mm).

This formulation limits unrealistically high erodibility values for highly silty soils and preserves consistency with the original nomograph behaviour, as discussed by Foster et al. (1981) and more recently by Corral-Pazos-de-Provens et al. (2023).

SM4.1.3	 Fixed structural and permeability parameters

Soil structure and permeability were assumed spatially uniform and fixed at b = 2 (fine granular structure) and c = 3 (moderate permeability). This parameterisation follows the pragmatic adaptation of the USLE recommended for Western European conditions, where spatially distributed measurements of soil structure and permeability classes are generally unavailable (Bollinne, 1985).

SM4.1.4	 Organic matter and coarse fragments

Soil organic matter content (a) was derived from the CARBIOSOL soil organic carbon map provided by the Service Public de Wallonie (SPW, 2022; Chartin et al., 2017) and converted to organic matter using standard conversion factors.

No coarse fragments (>2 mm) were observed within the cultivated soil layer. Their potential influence on erosion processes—through increased surface roughness, enhanced infiltration, and reduced detachment and transport—was therefore not explicitly accounted for in the K-factor estimation. This assumption is consistent with field observations and previous studies in loess environments (Poesen et al., 1994).

SM4.2	 Cover-management factor (C) and support practice factor (PUSLE)

The cover-management factor (C) quantifies the relative effect of land cover and crop management on soil erosion, with values ranging from 0 (fully protected surface) to 1 (bare soil).

Crop-specific C-factor values were assigned based on dominant land use and crop type following values reported by Swerts et al. (2020), representative of conventional agricultural practices in Belgium. Annual C-factor maps were generated at the plot scale for each year between 2015 and 2023 to account explicitly for interannual variability in crop rotations.

SM4.3	 Support practice factor

The support practice factor (PUSLE), accounting for erosion control measures such as contour farming or terracing, was uniformly set to 1, reflecting conventional management conditions without dedicated soil conservation practices.

SM4.4	 Topographic factor (LS)

SM4.4.1	 Specific contributing area

In grid-based implementations, slope length is replaced by the specific contributing area (As), which better captures flow convergence under complex terrain (Moore and Burch, 1986). It was computed as:

As=flow accumulation* cell size2cell size

SM4.4.2	 Slope length factor (L)

The slope length factor was calculated following the RUSLE formulation of Renard et al. (1997):

L=As22.13m with: m=β1+βand β=sin⁡θ/0.08963.0(sin⁡θ)0.8+0.56					
where θ is the local slope angle (radians).

SM4.4.3	 Slope steepness factor (S)

The slope steepness factor was computed following the RUSLE formulation of Renard et al. (1997) as:

S=10.8sin⁡θ+0.03,for slopes <9%16.8sin⁡θ-0.50,for slopes ≥9%

Slope gradients were derived from the 1-m DEM using the algorithm of Zevenbergen and Thorne (1987).

SM4.5	 Resolution dependency and implications for MUSLE

The grid-based LS formulation introduces an explicit dependence on DEM spatial resolution through flow accumulation and cell size, in contrast with the original MUSLE formulation where slope length was defined at the plot or hillslope scale. This resolution dependency may affect the magnitude of sediment yield estimates and the empirical sediment yield coefficient (α), which was originally calibrated under slope-length-based formulations.

SM4.6	Curve Number

The annual curve number values reported in Table SM6b correspond to the mean CN values. For each year, CN values were assigned according to the hydrological soil groups and the dominant land-cover/crop-management characteristics of the contributing area, and then averaged to obtain an annual catchment-scale mean CN (USDA-NRCS (2021)).

SM4.7	 Summary of fixed parameters and MUSLE inputs used in the study

Table SM5: Fixed parameters used in the study

| Parameter | Value | Description |
| --- | --- | --- |
| Soil structure class (b) | 2 | Fine granular |
| Soil permeability class (c) | 3 | Moderate |
| P-factor | 1 | No conservation practices |
| DEM resolution | 1 m | LiDAR-derived |
| Standard slope length | 22.13 m | USLE reference |

Table SM6: Parameters used for MUSLE computation

| Parameter | Value | Description |
| --- | --- | --- |
| L factor (Renard) | 40.63 | Slope length factor |
| S factor (Renard) | 0.44 | Slope steepness factor |
| Catchment area | 838,531 m² | Drainage basin surface |
| Mean slope | 0.038 | Average catchment slope |
| Mean erodibility factor | 0.058 | Soil erodibility |

Table SM6b: Annual mean Curve Number and C factor

| Year | CN mean | C factor |
| --- | --- | --- |
| 2015 | 72.71 | 0.28 |
| 2016 | 73.87 | 0.32 |
| 2017 | 72.87 | 0.29 |
| 2018 | 74.56 | 0.32 |
| 2019 | 73.34 | 0.29 |
| 2020 | 74.21 | 0.34 |
| 2021 | 72.85 | 0.28 |
| 2022 | 73.75 | 0.32 |
| 2023 | 72.89 | 0.28 |

SM5. Event-based observed runoff characteristics and corresponding λ estimates

Table SM7: Observed rainfall, runoff and peak flow, together with event-specific optimal initial abstraction ratio (λₒₚₜ) and their corresponding generalised estimates derived from K-means clustering (λkmc,diag), and linear regression (λcont,diag) for the 154 analysed events.

| Event ID |  | Observed |  |  |  |  | Diagnostic |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Rainfall (mm) | Runoff (mm) | Runoff (m3) | Peak flow (L.s-1) |  | λopt | λkmc,diag | λcont,diag |
| 0 |  | 18.00 | 1.19 | 993.87 | 163.04 |  | 0.071 | 0.058 | 0.119 |
| 1 |  | 29.00 | 2.67 | 2238.94 | 174.85 |  | 0.122 | 0.112 | 0.206 |
| 2 |  | 6.20 | 0.16 | 137.30 | 53.55 |  | 0.023 | 0.020 | 0.033 |
| 3 |  | 4.80 | 0.09 | 75.97 | 55.33 |  | 0.019 | 0.020 | 0.018 |
| 4 |  | 4.40 | 0.16 | 137.08 | 58.02 |  | 0.004 | 0.020 | 0.020 |
| 5 |  | 6.00 | 0.09 | 71.52 | 36.26 |  | 0.033 | 0.020 | 0.031 |
| 6 |  | 18.60 | 0.76 | 641.08 | 159.78 |  | 0.101 | 0.112 | 0.120 |
| 7 |  | 19.40 | 0.01 | 5.95 | 10.52 |  | 0.195 | 0.177 | 0.122 |
| 8 |  | 17.00 | 0.98 | 820.38 | 67.21 |  | 0.079 | 0.058 | 0.136 |
| 9 |  | 7.60 | 0.02 | 14.37 | 7.32 |  | 0.071 | 0.058 | 0.066 |
| 10 |  | 5.60 | 0.00 | 0.05 | 0.18 |  | 0.061 | 0.058 | 0.053 |
| 11 |  | 24.40 | 0.06 | 50.45 | 26.84 |  | 0.245 | 0.283 | 0.193 |
| 12 |  | 5.80 | 0.03 | 22.81 | 16.47 |  | 0.047 | 0.058 | 0.045 |
| 13 |  | 17.60 | 0.19 | 157.35 | 22.57 |  | 0.149 | 0.177 | 0.143 |
| 14 |  | 4.80 | 0.01 | 9.64 | 11.58 |  | 0.042 | 0.058 | 0.041 |
| 15 |  | 7.20 | 0.13 | 110.90 | 29.50 |  | 0.041 | 0.058 | 0.059 |
| 16 |  | 8.40 | 0.06 | 48.87 | 43.43 |  | 0.068 | 0.058 | 0.066 |
| 17 |  | 4.00 | 0.05 | 40.27 | 15.78 |  | 0.021 | 0.020 | 0.035 |
| 18 |  | 6.20 | 0.02 | 18.32 | 17.88 |  | 0.053 | 0.058 | 0.056 |
| 19 |  | 6.20 | 0.05 | 39.15 | 13.15 |  | 0.046 | 0.058 | 0.058 |
| 20 |  | 6.80 | 0.12 | 99.52 | 16.93 |  | 0.039 | 0.020 | 0.063 |
| 21 |  | 18.20 | 0.51 | 425.59 | 88.12 |  | 0.125 | 0.112 | 0.109 |
| 22 |  | 2.60 | 0.02 | 17.62 | 13.56 |  | 0.014 | 0.020 | 0.029 |
| 23 |  | 10.80 | 0.27 | 226.07 | 62.07 |  | 0.064 | 0.058 | 0.077 |
| 24 |  | 10.00 | 0.01 | 5.87 | 4.76 |  | 0.102 | 0.112 | 0.084 |
| 25 |  | 3.60 | 0.00 | 0.55 | 0.89 |  | 0.037 | 0.020 | 0.036 |
| 26 |  | 5.00 | 0.01 | 7.60 | 5.50 |  | 0.046 | 0.058 | 0.049 |
| 27 |  | 8.00 | 0.03 | 27.91 | 17.00 |  | 0.070 | 0.058 | 0.066 |
| 28 |  | 5.60 | 0.08 | 64.15 | 6.35 |  | 0.030 | 0.020 | 0.033 |
| 29 |  | 10.40 | 0.10 | 83.35 | 5.98 |  | 0.077 | 0.058 | 0.068 |
| 30 |  | 9.00 | 0.36 | 298.94 | 16.78 |  | 0.032 | 0.020 | 0.060 |
| 31 |  | 6.00 | 0.05 | 38.66 | 5.81 |  | 0.041 | 0.058 | 0.037 |
| 32 |  | 6.20 | 0.01 | 11.88 | 5.12 |  | 0.053 | 0.058 | 0.029 |
| 33 |  | 5.00 | 0.03 | 25.87 | 5.58 |  | 0.035 | 0.020 | 0.028 |
| 34 |  | 1.80 | 0.02 | 19.08 | 5.54 |  | 0.003 | 0.020 | 0.006 |
| 35 |  | 4.60 | 0.00 | 1.50 | 4.99 |  | 0.044 | 0.058 | 0.025 |
| 36 |  | 10.00 | 0.01 | 4.59 | 5.31 |  | 0.098 | 0.112 | 0.066 |
| 37 |  | 28.91 | 0.07 | 62.85 | 21.09 |  | 0.304 | 0.283 | 0.245 |
| 38 |  | 6.23 | 0.15 | 121.58 | 14.52 |  | 0.030 | 0.020 | 0.075 |
| 39 |  | 4.93 | 0.13 | 109.57 | 10.89 |  | 0.017 | 0.020 | 0.066 |
| 40 |  | 26.82 | 0.08 | 65.76 | 85.59 |  | 0.279 | 0.283 | 0.230 |
| 41 |  | 18.85 | 0.03 | 24.30 | 5.80 |  | 0.186 | 0.177 | 0.138 |
| 42 |  | 8.75 | 0.03 | 22.50 | 5.80 |  | 0.078 | 0.058 | 0.065 |
| 43 |  | 9.70 | 0.00 | 2.04 | 6.80 |  | 0.100 | 0.112 | 0.062 |
| 44 |  | 12.91 | 0.14 | 114.24 | 97.30 |  | 0.101 | 0.112 | 0.072 |
| 45 |  | 15.55 | 0.02 | 17.16 | 18.00 |  | 0.153 | 0.177 | 0.120 |
| 46 |  | 34.88 | 0.31 | 257.10 | 69.90 |  | 0.318 | 0.283 | 0.261 |
| 47 |  | 27.40 | 0.11 | 90.89 | 28.20 |  | 0.262 | 0.283 | 0.221 |
| 48 |  | 4.86 | 0.01 | 9.77 | 7.89 |  | 0.041 | 0.058 | 0.029 |
| 49 |  | 7.08 | 0.00 | 1.45 | 4.83 |  | 0.072 | 0.058 | 0.055 |
| 50 |  | 3.27 | 0.03 | 28.41 | 7.20 |  | 0.016 | 0.020 | 0.027 |
| 51 |  | 4.60 | 0.00 | 3.51 | 6.50 |  | 0.043 | 0.058 | 0.036 |
| 52 |  | 4.81 | 0.02 | 18.39 | 9.40 |  | 0.037 | 0.020 | 0.036 |
| 53 |  | 11.54 | 0.06 | 46.68 | 12.10 |  | 0.100 | 0.112 | 0.082 |
| 54 |  | 13.80 | 0.01 | 11.25 | 5.80 |  | 0.137 | 0.112 | 0.100 |
| 55 |  | 7.09 | 0.03 | 22.47 | 8.99 |  | 0.063 | 0.058 | 0.066 |
| 56 |  | 18.79 | 0.01 | 5.89 | 5.15 |  | 0.204 | 0.177 | 0.159 |
| 57 |  | 18.13 | 0.04 | 33.26 | 6.15 |  | 0.184 | 0.177 | 0.149 |
| 58 |  | 4.62 | 0.00 | 1.64 | 5.48 |  | 0.048 | 0.058 | 0.055 |
| 59 |  | 16.22 | 0.32 | 267.04 | 12.88 |  | 0.122 | 0.112 | 0.137 |
| 60 |  | 5.85 | 0.28 | 236.12 | 17.95 |  | 0.008 | 0.020 | 0.063 |
| 61 |  | 4.83 | 0.01 | 6.08 | 5.15 |  | 0.046 | 0.058 | 0.046 |
| 62 |  | 4.49 | 0.01 | 4.35 | 4.83 |  | 0.043 | 0.058 | 0.053 |
| 63 |  | 8.27 | 0.11 | 93.57 | 4.51 |  | 0.052 | 0.058 | 0.049 |
| 64 |  | 3.97 | 0.07 | 61.37 | 3.30 |  | 0.014 | 0.020 | 0.016 |
| 65 |  | 9.15 | 0.15 | 125.33 | 16.65 |  | 0.056 | 0.058 | 0.053 |
| 66 |  | 10.77 | 0.12 | 97.82 | 3.30 |  | 0.078 | 0.058 | 0.072 |
| 67 |  | 13.99 | 0.37 | 309.45 | 23.41 |  | 0.083 | 0.058 | 0.093 |
| 68 |  | 7.80 | 0.38 | 319.48 | 12.47 |  | 0.017 | 0.020 | 0.047 |
| 69 |  | 7.04 | 0.38 | 315.40 | 8.25 |  | 0.009 | 0.020 | 0.044 |
| 70 |  | 12.99 | 1.07 | 898.36 | 121.21 |  | 0.025 | 0.020 | 0.078 |
| 71 |  | 12.48 | 0.22 | 186.99 | 3.90 |  | 0.082 | 0.058 | 0.081 |
| 72 |  | 7.25 | 0.20 | 169.06 | 4.51 |  | 0.029 | 0.020 | 0.044 |
| 73 |  | 6.12 | 0.16 | 136.65 | 3.60 |  | 0.022 | 0.020 | 0.038 |
| 74 |  | 22.59 | 0.29 | 246.77 | 3.90 |  | 0.181 | 0.177 | 0.151 |
| 75 |  | 2.25 | 0.01 | 6.95 | 1.93 |  | 0.014 | 0.020 | 0.011 |
| 76 |  | 6.63 | 0.00 | 0.58 | 1.93 |  | 0.067 | 0.058 | 0.026 |
| 77 |  | 5.52 | 0.02 | 13.79 | 2.19 |  | 0.045 | 0.058 | 0.032 |
| 78 |  | 9.21 | 0.14 | 114.86 | 4.83 |  | 0.059 | 0.058 | 0.057 |
| 79 |  | 7.82 | 0.16 | 135.16 | 10.12 |  | 0.040 | 0.058 | 0.038 |
| 80 |  | 7.55 | 0.28 | 235.22 | 78.59 |  | 0.024 | 0.020 | 0.037 |
| 81 |  | 14.37 | 0.49 | 407.07 | 170.92 |  | 0.078 | 0.058 | 0.093 |
| 82 |  | 3.35 | 0.03 | 25.61 | 3.30 |  | 0.017 | 0.020 | 0.012 |
| 83 |  | 35.31 | 3.33 | 2793.02 | 130.39 |  | 0.167 | 0.177 | 0.252 |
| 84 |  | 37.01 | 2.72 | 2277.07 | 177.04 |  | 0.207 | 0.177 | 0.271 |
| 85 |  | 5.95 | 0.20 | 169.01 | 7.89 |  | 0.016 | 0.020 | 0.033 |
| 86 |  | 11.39 | 0.52 | 435.15 | 61.46 |  | 0.043 | 0.058 | 0.063 |
| 87 |  | 19.24 | 1.48 | 1241.87 | 267.69 |  | 0.070 | 0.058 | 0.107 |
| 88 |  | 45.51 | 2.97 | 2491.10 | 310.03 |  | 0.287 | 0.283 | 0.336 |
| 89 |  | 10.85 | 0.36 | 302.55 | 71.74 |  | 0.051 | 0.058 | 0.067 |
| 90 |  | 11.78 | 0.31 | 261.19 | 54.45 |  | 0.065 | 0.058 | 0.067 |
| 91 |  | 10.88 | 0.44 | 372.04 | 106.74 |  | 0.044 | 0.058 | 0.071 |
| 92 |  | 4.76 | 0.12 | 98.76 | 14.11 |  | 0.014 | 0.020 | 0.030 |
| 93 |  | 7.57 | 0.47 | 392.35 | 103.36 |  | 0.007 | 0.020 | 0.034 |
| 94 |  | 3.08 | 0.07 | 59.92 | 3.60 |  | 0.005 | 0.020 | 0.011 |
| 95 |  | 5.26 | 0.07 | 57.55 | 3.60 |  | 0.028 | 0.020 | 0.028 |
| 96 |  | 4.41 | 0.08 | 68.37 | 4.83 |  | 0.017 | 0.020 | 0.024 |
| 97 |  | 8.40 | 0.08 | 64.17 | 3.01 |  | 0.060 | 0.058 | 0.049 |
| 98 |  | 3.40 | 0.02 | 18.65 | 2.19 |  | 0.020 | 0.020 | 0.016 |
| 99 |  | 32.20 | 0.25 | 213.56 | 16.65 |  | 0.287 | 0.283 | 0.233 |
| 100 |  | 4.60 | 0.05 | 43.55 | 2.46 |  | 0.025 | 0.020 | 0.026 |
| 101 |  | 3.40 | 0.03 | 27.54 | 2.46 |  | 0.017 | 0.020 | 0.021 |
| 102 |  | 2.80 | 0.03 | 25.66 | 2.46 |  | 0.011 | 0.020 | 0.014 |
| 103 |  | 4.47 | 0.03 | 21.95 | 2.46 |  | 0.030 | 0.020 | 0.024 |
| 104 |  | 3.79 | 0.04 | 35.28 | 2.73 |  | 0.019 | 0.020 | 0.016 |
| 105 |  | 5.51 | 0.02 | 16.26 | 2.19 |  | 0.044 | 0.058 | 0.031 |
| 106 |  | 3.30 | 0.06 | 47.06 | 3.01 |  | 0.010 | 0.020 | 0.018 |
| 107 |  | 3.36 | 0.03 | 25.96 | 2.46 |  | 0.017 | 0.020 | 0.017 |
| 108 |  | 3.76 | 0.02 | 16.95 | 2.46 |  | 0.025 | 0.020 | 0.018 |
| 109 |  | 7.90 | 0.10 | 80.66 | 3.01 |  | 0.051 | 0.058 | 0.049 |
| 110 |  | 7.75 | 0.08 | 63.06 | 2.73 |  | 0.053 | 0.058 | 0.047 |
| 111 |  | 15.89 | 0.23 | 195.11 | 5.81 |  | 0.117 | 0.112 | 0.106 |
| 112 |  | 18.69 | 0.22 | 186.68 | 20.18 |  | 0.148 | 0.177 | 0.123 |
| 113 |  | 8.06 | 0.11 | 92.41 | 6.15 |  | 0.050 | 0.058 | 0.049 |
| 114 |  | 10.66 | 0.11 | 91.25 | 25.78 |  | 0.083 | 0.058 | 0.081 |
| 115 |  | 14.01 | 0.57 | 476.57 | 20.18 |  | 0.072 | 0.058 | 0.110 |
| 116 |  | 16.02 | 1.16 | 969.81 | 85.59 |  | 0.058 | 0.058 | 0.126 |
| 117 |  | 13.54 | 0.09 | 74.04 | 10.89 |  | 0.118 | 0.112 | 0.107 |
| 118 |  | 4.24 | 0.00 | 0.58 | 1.93 |  | 0.044 | 0.058 | 0.042 |
| 119 |  | 7.00 | 0.19 | 162.78 | 53.30 |  | 0.030 | 0.020 | 0.052 |
| 120 |  | 3.80 | 0.05 | 42.20 | 5.81 |  | 0.018 | 0.020 | 0.042 |
| 121 |  | 18.64 | 1.04 | 869.08 | 263.45 |  | 0.093 | 0.112 | 0.123 |
| 122 |  | 15.18 | 0.07 | 58.59 | 12.07 |  | 0.140 | 0.112 | 0.114 |
| 123 |  | 19.50 | 1.51 | 1269.70 | 251.68 |  | 0.078 | 0.058 | 0.128 |
| 124 |  | 14.70 | 1.13 | 950.17 | 117.72 |  | 0.044 | 0.058 | 0.116 |
| 125 |  | 13.75 | 0.02 | 17.42 | 2.73 |  | 0.137 | 0.112 | 0.109 |
| 126 |  | 7.70 | 0.02 | 19.02 | 30.68 |  | 0.069 | 0.058 | 0.060 |
| 127 |  | 12.05 | 0.05 | 38.56 | 17.08 |  | 0.110 | 0.112 | 0.090 |
| 128 |  | 13.36 | 0.08 | 66.25 | 3.30 |  | 0.118 | 0.112 | 0.106 |
| 129 |  | 8.83 | 0.02 | 18.70 | 2.19 |  | 0.082 | 0.058 | 0.071 |
| 130 |  | 11.32 | 0.05 | 45.02 | 2.46 |  | 0.101 | 0.112 | 0.086 |
| 131 |  | 7.50 | 0.07 | 55.04 | 2.19 |  | 0.056 | 0.058 | 0.063 |
| 132 |  | 12.72 | 0.01 | 7.74 | 2.46 |  | 0.125 | 0.112 | 0.080 |
| 133 |  | 14.04 | 0.09 | 79.37 | 4.20 |  | 0.116 | 0.112 | 0.089 |
| 134 |  | 7.57 | 0.18 | 153.95 | 43.24 |  | 0.035 | 0.020 | 0.047 |
| 135 |  | 20.56 | 0.12 | 104.29 | 61.46 |  | 0.181 | 0.177 | 0.146 |
| 136 |  | 8.16 | 0.10 | 87.78 | 47.65 |  | 0.053 | 0.058 | 0.041 |
| 137 |  | 8.71 | 0.37 | 306.36 | 101.34 |  | 0.028 | 0.020 | 0.039 |
| 138 |  | 7.37 | 0.00 | 1.16 | 1.93 |  | 0.074 | 0.058 | 0.038 |
| 139 |  | 11.32 | 0.01 | 7.15 | 3.30 |  | 0.110 | 0.112 | 0.066 |
| 140 |  | 11.35 | 0.01 | 12.26 | 4.51 |  | 0.108 | 0.112 | 0.072 |
| 141 |  | 11.55 | 0.01 | 7.17 | 3.30 |  | 0.113 | 0.112 | 0.073 |
| 142 |  | 5.57 | 0.02 | 15.69 | 4.83 |  | 0.045 | 0.058 | 0.024 |
| 143 |  | 13.42 | 0.09 | 78.98 | 11.28 |  | 0.110 | 0.112 | 0.091 |
| 144 |  | 12.93 | 0.04 | 30.54 | 3.30 |  | 0.117 | 0.112 | 0.085 |
| 145 |  | 7.83 | 0.06 | 46.61 | 10.12 |  | 0.058 | 0.058 | 0.047 |
| 146 |  | 11.79 | 0.08 | 68.91 | 18.84 |  | 0.095 | 0.112 | 0.072 |
| 147 |  | 6.08 | 0.03 | 22.22 | 4.51 |  | 0.047 | 0.058 | 0.036 |
| 148 |  | 14.21 | 0.22 | 185.69 | 22.01 |  | 0.101 | 0.112 | 0.092 |
| 149 |  | 13.01 | 0.06 | 53.37 | 3.60 |  | 0.111 | 0.112 | 0.087 |
| 150 |  | 18.04 | 0.92 | 775.44 | 88.82 |  | 0.087 | 0.112 | 0.117 |
| 151 |  | 7.50 | 0.44 | 364.74 | 28.20 |  | 0.009 | 0.020 | 0.045 |
| 152 |  | 10.31 | 0.90 | 754.90 | 38.94 |  | 0.007 | 0.020 | 0.067 |
| 153 |  | 5.79 | 0.56 | 469.36 | 38.94 |  | 0.000 | 0.020 | 0.024 |

## SM6.	Generalisation of runoff initiation

## This section provides the complete formulation and evaluation of the continuous regression models used to generalise the event-specific initial abstraction ratio from hydro-meteorological descriptors. It presents the simple and interaction-based linear models, the power-law, exponential, and Michaelis–Menten-type formulations, and the criteria used to compare their calibration, predictive performance, and parsimony. LASSO-regularised variants are included as robustness checks. Table SM8 reports the estimated model coefficients and their statistical support, while Table SM9 compares the calibration and cross-validated performance of the candidate and regularised formulations.

### SM6.1	Continuous regression-based formulations tested for λ prediction

As a complement to the regime-based generalisation strategy, several continuous regression-based formulations were tested to predict event-scale values of λ from hydro-meteorological descriptors. The explanatory variables considered were total rainfall depth (P), potential maximum retention (S), rainfall kinetic energy (E), and maximum 30-minute rainfall intensity (I30).

The following candidate formulations were evaluated.

Simple multiple linear model – LM (simple)

λ=a+b P+c S+d E+e I30

Extended linear model with pairwise interactions – LM (interaction)

λ=a+b* P+c* S+d* E+e* I30+f(P*S)+g(P*E)+h(P*I30)+i(S*E)+j(S*I30)+k(E*I30)

Multiplicative power-law model

λ=a* Pb*Sc*Ed*I30e

Exponential log-linear model

λ=exp⁡(a+b* P+c* S+d* E+e* I30)

Michaelis–Menten-type model

λ=a* Pb+P+c* S+d* E+e* I30

where a, b, c, d, and e are fitted parameters, and f to k denote the coefficients associated with pairwise interaction terms in the extended linear model.

## SM6.2	 Model comparison and regularisation

Continuous models were compared using calibration performance, cross-validation performance, and corrected Akaike Information Criterion (AICc):

AICc=AIC+2k(k+1)n-k-1

where k is the number of fitted parameters and n is the number of events.

LASSO-regularised variants were tested as robustness checks:

mini=1n(λopt,i-λi)2+θj=1p∣βj∣

where θ is the regularisation parameter and βj are regression coefficients. LASSO results were used only to assess the robustness of variable selection and model ranking.

Table SM8: Estimated coefficients, associated p-values, and coefficients of determination (R2, adjusted R2) for the candidate continuous formulations tested for event-scale λ cont,diag: simple linear model (LM), linear interaction model, power-law model, exponential model, and Michaelis–Menten model. Significance codes are reported as follows: ⋅p<0.10, * p<0.05, ** p<0.01, *** p<0.001.

Table SM9: Predictive performance of the candidate continuous formulations for event-scale λ estimation, including unregularised and LASSO-regularised variants. Calibration and 5-fold cross-validation results are reported using NSE, KGE, PBIAS and RRMSE. Model parsimony is additionally assessed using AIC and AICc.

SM7. Sensitivity analyses of the predictive formulations

Table SM10: Sensitivity of the Random Forest-assigned five-regime formulation to cumulative antecedent rainfall. Values are means across ten repetitions of fivefold cross-validation. The first row corresponds to the predictor set used in the main analysis, comprising rainfall depth, annual potential maximum retention, rainfall kinetic energy, maximum 30-min rainfall intensity, and seasonal timing, without an antecedent-rainfall variable. ΔNSE represents the difference relative to this predictor set. The 95% intervals were calculated from repetition-specific paired differences and are reported descriptively because the cross-validation training samples overlapped.

|  | Predictor configuration | NSE | ΔNSE [descriptive 95% interval] | KGE | PBIAS (%) | RRMSE (%) | RF accuracy | Macro-F1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Predictor set used in the main analysis, without antecedent rainfall | 0.205 | — | 0.552 | -17.649 | 179.452 | 0.643 | 0.584 |  |
|  | Antecedent rainfall, 3 days | 0.249 | +0.044 [−0.008; 0.096] | 0.574 | 13.941 | 174.458 | 0.629 | 0.577 |  |
|  | Antecedent rainfall, 5 days | 0.047 | −0.158 [−0.216; −0.101] | 0.496 | 20.197 | 196.711 | 0.618 | 0.555 |  |
|  | Antecedent rainfall, 7 days | 0.120 | −0.085 [−0.162; −0.009] | 0.508 | 20.359 | 188.722 | 0.624 | 0.558 |  |
|  | Antecedent rainfall, 10 days | 0.188 | −0.017 [−0.091; 0.057] | 0.583 | 14.093 | 180.681 | 0.635 | 0.570 |  |

References (Supplementary Material)

Baltas, E. A., Dervos, N. A., Mimikou, M. A. (2007). Technical Note: Determination of the SCS initial abstraction ratio in an experimental watershed in Greece. Hydrology and Earth System Sciences, 11, 1825–1829. https://doi.org/10.5194/hess-11-1825-2007.

Bollinne, A., Florins, P., Hecq, P., Homerin, D., Renard, V., Wolfs, J.-L. (1984). Étude de l’énergie des pluies en climat tempéré océanique d’Europe Atlantique. Zeitschrift für Geomorphologie,  Supplement 49: 27–35. https://www.schweizerbart.de/publications/detail/isbn/9783443210496/Z_Geomorph_Suppl_Bd_49_alt_Pres

Bollinne, A. (1985). Adjusting the universal soil loss equation for use in Western Europe. In Soil erosion and conservation. 206-213. Edited by A. El-Swaify, W. C. Woldenhauer and A. Lo. Soil Conservation Society of America, Ankeny, Iowa.

Chartin, C., Stevens, A., Goidts, E., Krüger, I., Carnol, M., van Wesemael, B. (2017). Mapping Soil Organic Carbon stocks and estimating uncertainties at the regional scale following a legacy sampling strategy (Southern Belgium, Wallonia). Geoderma Regional, 9, 73-86. http://dx.doi.org/10.1016/j.geodrs.2016.12.006

Corral-Pazos-de-Provens, E., Rapp-Arrarás, Í., Domingo-Santos, J.M., (2023). The USLE soil erodibility nomograph revisited. Int. Soil Water Conserv. Res. 11, 1–13. https://doi.org/10.1016/j.iswcr.2022.07.001

Foster, G. R., McCool, D. K., Renard, K. G., Moldenhauer, W. C. (1981). Conversion of the universal soil loss equation to SI metric units. Journal of Soil and Water Conservation, 36(6), 355–359. https://doi.org/10.1080/00224561.1981.12436140

Hawkins, R. H., Ward, T. J., Woodward, D. E., Van Mullem, J. A. (2008). Curve Number Hydrology: State of the Practice. Reston, VA: American Society of Civil Engineers (ASCE). https://doi.org/10.1061/9780784410042

Jain MK, Mishra SK, Suresh Babu P, Venugopal K, Singh VP. (2006). Enhanced Runoff Curve Number Model Incorporating Storm Duration and a Nonlinear Ia -S Relation. Journal of Hydrologic Engineering 11 (6): 631–635. https://doi.org/10.1061/(ASCE)1084-0699(2006)11:6(631)

McGregor, K. C., Bingner, R. L., Bowie, A. J., Foster, G. R. (1995). Erosivity index values for northern Mississippi. Transactions of the ASAE, 38(4), 1039–1047. https://doi.org/10.13031/2013.27921

Mishra, S. K., Singh, V. P. (2004). Long-term hydrological simulation based on the Soil Conservation Service curve number. Hydrological Processes, 18(7), 1291–1313. https://doi.org/10.1002/hyp.1344.

Moore, I. D., Burch, G. J. (1986). Physical basis of the length–slope factor in the Universal Soil Loss Equation. Soil Science Society of America Journal, 50(5), 1294–1298. https://doi.org/10.2136/sssaj1986.03615995005000050042x

Poesen, J., Torri, D., Bunte, K. (1994). Effects of rock fragments on soil erosion by water at different spatial scales. Catena, 23(1–2), 141–166. https://doi.org/10.1016/0341-8162(94)90058-2

Renard, K. G., Foster, G. R., Weesies, G. A., McCool, D. K., Yoder, D. C. (1997). Predicting Soil Erosion by Water: A Guide to Conservation Planning with the Revised Universal Soil Loss Equation (RUSLE) USDA Handbook 703, pp 104-110. ISBN: 0-16-048938-5

Service Public de Wallonie (SPW), (2022). CARBIOSOL - Carbone organique total des Sols – Série. https://geoportail.wallonie.be/catalogue/47e4ea34-fe00-4712-b795-4a85fdab7dd7.html

Swerts, M., Broekaert, S., Deproost, P., Oorts, K., 2020. Bodemerosierisico-indicator Vlaanderen (2008-2019). Department Omgeving. https://archief.algemeen.omgeving.vlaanderen.be/xmlui/bitstream/handle/acd/271600/Eindrapport%20bodemerosierisicoindicator2008-2019.pdf

U.S. Department of Agriculture, Natural Resources Conservation Service (USDA-NRCS) (2021). Estimating Runoff Volume and Peak Discharge. National Engineering Handbook, Title 210, Part 650, Chapter 2, 2nd edition, Figure 2-7. URL : https://directives.nrcs.usda.gov/sites/default/files2/1712930818/31754.pdf

Wischmeier, W. H., Johnson, C. B., Cross, B. V. (1971). A soil erodibility nomograph for farmland and construction sites. Journal of Soil and Water Conservation, 26(5), 189-193.

Woodward, D. E., Hawkins, R. H., Jiang, R., Hjelmfelt, A. T., Van Mullem, J. A., Quan, Q. D. (2003). Runoff curve number method: Examination of the initial abstraction ratio. In World Water & Environmental Resources Congress 2003. American Society of Civil Engineers. https://doi.org/10.1061/40685(2003)308.

Zevenbergen, L. W., Thorne, C. R. (1987). Quantitative analysis of land surface topography. Earth Surface Processes and Landforms, 12(1), 47–56. https://doi.org/10.1002/esp.3290120107